Justification:

- Addition of a formal definitions table to improve clarity and reduce ambiguity of commonly used terms.

- Clarify that, unless specified as part of a device type’s definition, each node on an RV-C network should claim exactly one source address.

Original Section:

3 Intermediate Layers

3.1 Data Frame Structure

Messages shall use only CAN data frames in extended frame format with a DLC (Data Length) of 8. CAN remote frames shall not be used. Figure 3.1 shows the structure of the CAN data frame in extended frame format.

Figure 3.1 CAN data frame structure

3.2 Network and Transport Layers

The network, transport, and application layer protocols shall use the base and extended ID field as well as the data field of the CAN data frame. The network, transport, and application layer message structure is defined in 3.2

Table 3.2: Structure of the network, transport, and application layer message

| DLL fields |  | RV-C name (abbreviation) | Description |
| --- | --- | --- | --- |
| Name | Bit |  |  |
| Base ID | 28 to 26 | Priority | 111b – Lowest priority 000b – Highest priority |
|  | 25 | Reserved | Always 0b |
|  | 24 to 18 | Data Group Number - High | Identifies how the data packet should be parsed, possibly in combination with the DGN-Low |
| Extended ID | 17 and 16 |  |  |
|  | 15 to 8 | Data Group Number - Low | Either determines the target node for the message, or with the DGN-High determines how the data packet should be parsed. |
|  | 7 to 0 | Source address (SA) | Shall be unique for each node |
| Data | 64 to 0 | Data | Defined in detail in the RV-C application profile specification |

The priority bits and SA bits are essential for bus arbitration. When two RV-C nodes attempt to transmit simultaneously, the priority bits determine which message will get on the bus first. And as all RV-C nodes have a unique source address, the SA serves as the transmission tiebreaker-of-last-resort. In general, receiving nodes are to ignore these fields. These fields are not to be used for any other purpose, and in particular they are not to be parsed as meaningful data, except as specifically described in certain specialized applications.

The network, transport, and application layer protocol demands that all RV-C nodes respond to certain messages. All messages on the network shall conform to this specification.

3.2.1 Source addresses

3.2.1.1 Introduction

Every RV-C node shall have a unique source address to serve as a final tiebreaker during bus arbitration. The source address does not fully identify the RV-C node, and shall not be used by other RV-C nodes to interpret data from that node, except in matters of address claiming, proprietary messaging, and diagnostics.

Source addresses shall be assigned in one of two ways. RV-C node designers may choose to "hard wire" a standard address for the particular RV-C node type as defined in clause 3.2.1.2. Integrators shall take care to ensure that no two RV-C nodes using this technique are installed with the very same address.

A designer seeking greater flexibility may use the address claiming procedure, which dynamically assigns an SA when the RV-C node is powered up. The procedure is described in clause 3.3.

Proposed Changes in RED:

3 Intermediate Layers

3.1 Data Frame Structure

Messages shall use only CAN data frames in extended frame format with a DLC (Data Length) of 8. CAN remote frames shall not be used. Figure 3.1 shows the structure of the CAN data frame in extended frame format.

Figure 3.1 CAN data frame structure

3.2 Network and Transport Layers

Table 3.2a defines terms commonly used throughout the RV-C protocol specification.

Table 3.2a: Definitions

| Term | Definition |
| --- | --- |
| Node or “RV-C Node” or “CAN Node” | A microprocessor that sends and receives data on an RV-C bus. More technically, a CAN 2.0B master node consisting of a transceiver, controller, and microprocessor/microcontroller to implement the RV-C protocol. |
| Source Address or “SA” | A unique identifier for each node on an RV-C network. Source addresses may be hard coded by a device, or claimed dynamically on startup per section 3.3 of this specification. A node maintains a single source address regardless of the number of functions or device types it supports. |
| Device Type or “Type” | A set of functions or capabilities possessed by a node e.g. “awning”, “inverter”, or “water heater”. Each device type has a set of RV-C messages associated with it as defined in section 6 of this specification; e.g. a node supporting the “awning” device type might broadcast/receive RV-C messages “AWNING_COMMAND” and/or “AWNING_STATUS”. A node may support multiple device types or multiple instances of a device type (e.g. “awning 1” and “awning 2”.) |
| DSA or “Default Source Address” | An identifier associated with each device type as defined in Table 7.2. The DSA is included when a node broadcasts its diagnostic message (DM_RV); a node supporting multiple device types broadcasts the DM_RV for each of its device types with the correct DSA in each DM_RV message. |

The network, transport, and application layer protocols shall use the base and extended ID field as well as the data field of the CAN data frame. The network, transport, and application layer message structure is defined in table 3.2b.

Table 3.2b: Structure of the network, transport, and application layer message

| DLL fields |  | RV-C name (abbreviation) | Description |
| --- | --- | --- | --- |
| Name | Bit |  |  |
| Base ID | 28 to 26 | Priority | 111b – Lowest priority 000b – Highest priority |
|  | 25 | Reserved | Always 0b |
|  | 24 to 18 | Data Group Number - High | Identifies how the data packet should be parsed, possibly in combination with the DGN-Low |
| Extended ID | 17 and 16 |  |  |
|  | 15 to 8 | Data Group Number - Low | Either determines the target node for the message, or with the DGN-High determines how the data packet should be parsed. |
|  | 7 to 0 | Source address (SA) | Shall be unique for each node |
| Data | 64 to 0 | Data | Defined in detail in the RV-C application profile specification |

The priority bits and SA bits are essential for bus arbitration. When two RV-C nodes attempt to transmit simultaneously, the priority bits determine which message will get on the bus first. And as all RV-C nodes have a unique source address, the SA serves as the transmission tiebreaker-of-last-resort. In general, receiving nodes are to ignore these fields. These fields are not to be used for any other purpose, and in particular they are not to be parsed as meaningful data, except as specifically described in certain specialized applications.

The network, transport, and application layer protocol demands that all RV-C nodes respond to certain messages. All messages on the network shall conform to this specification.

3.2.1 Source addresses

3.2.1.1 Introduction

Every RV-C node shall have a unique source address to serve as a final tiebreaker during bus arbitration. The source address does not fully identify the RV-C node, and shall not be used by other RV-C nodes to interpret data from that node, except in matters of address claiming, proprietary messaging, and diagnostics. Unless otherwise noted, each node on an RV-C network should claim exactly one source address, even if it implements more than one device type or instance of a device type.

Source addresses shall be assigned in one of two ways. RV-C node designers may choose to "hard wire" a standard address for the particular RV-C node type as defined in clause 3.2.1.2. Integrators shall take care to ensure that no two RV-C nodes using this technique are installed with the very same address.

A designer seeking greater flexibility may use the address claiming procedure, which dynamically assigns an SA when the RV-C node is powered up. The procedure is described in clause 3.3.