Justification:

- Clarify that, unless specified as part of a device type’s definition, each node on an RV-C network should claim exactly one source address.

- Give details about a commonly used address claiming practice.

- Clarify instances in which higher priority nodes are to take the addresses of lower priority nodes.

- Deprecate “Node Instance” and “Function Instance” from ADDRESS_CLAIM table 3.3.3.

Original Section:

3.3.1 Introduction

The RV-C node that has no assigned SA address may use the node claiming procedure to get an unique SA address.

3.3.2 Source Address claiming

The RV-C node requesting dynamically an SA begins with an address request message. Every RV-C node shall be able to respond to the address request message with an address claimed message. RV-C nodes that are "hard wired" to an SA shall respond to the address request in order to prevent a dynamically addressed RV-C node from taking its address. A dynamically addressed RV-C node will try addresses until it finds one unclaimed. Once it finds an unclaimed address, it sends the address claimed message.

Table 3.3.2a defines the address request message.

Table 3.3.2a - Address request message

| Attribute | Value definition |
| --- | --- |
| DGN | EA00h |
| DGN-High | EAh |
| DGN-Low | Address desired |
| Priority: | 6 |
| Data length | 8 bytes |
| Broadcast gap | As needed |
| Source address | 254 (no address has been claimed) |
| Data 0 | 0 |
| Data 1 | 238 |
| Data 2 | 0 |
| NOTE It will be seen that that this is just an ordinary request for DGN EE00h, but directed at a particular address. |  |

Table 3.3.2b defines the address claimed message.

Table 3.3.2b - Address claimed

| Attribute | Value definition |
| --- | --- |
| DGN | EE00h |
| DGN-High | EEh |
| DGN-Low | 00h |
| Piority | 6 |
| Broadcast gap | On request only |
| Source address | Assigned SA |
| Data 0 to 7 | "ADDRESS_CLAIM" (8-byte identifier, see clause 3.3.3) |

In order to reduce the possibility of mis-configured RV-C nodes (ending up with the very same SA), RV-C nodes used in the same network should try to claim different SAs. RV-C nodes should start with the address given for its RV-C node type in Table 7.2. The RV-C node shall not attempt to claim an address that might be used by a statically addressed RV-C node. If an address is already claimed, the RV-C node shall try the next address lower, and thus count downwards.

The ADDRESS_CLAIM field as described in Section 3.3.3 is used to determine the priority of RV-C nodes in address claiming. RV-C nodes with a higher priority ADDRESS_CLAIM field may "take" an address from a lower priority RV-C node. Statically addressed RV-C nodes may keep their address. Dynamically addressed RV-C nodes shall yield to static RV-C nodes, or to dynamic RV-C nodes with a higher priority ADDRESS_CLAIM field .

An RV-C node with dynamic addresses shall monitor network traffic and if they see another RV-C node using its address and it has a higher priority ADDRESS_CLAIM field it shall automatically run through the procedure and claim a new address.

3.3.3 ADDRESS_CLAIM field

The ADDRESS_CLAIM field is used as a method of uniquely identifying RV-C nodes during the address claiming procedure and for network troubleshooting. When two or more nodes attempt to claim the same source address, the ADDRESS_CLAIM field is used for arbitration. The ADDRESS_CLAIM field should be treated as an eight byte value with the “Arbitrary Address Capable” bit being the most significant bit. This ensures statically addressed nodes will always have the highest priority.

The ADDRESS_CLAIM field is defined in Table 3.3.3.

Table 3.3.3: ADDRESS_CLAIM field

| Byte | Bit | Name | Description |
| --- | --- | --- | --- |
| 0 | 0 to 7 | Serial number (LSB) | Optional. Required if multiple nodes from the same manufacturer may be present on the network. |
| 1 | 0 to 7 | Serial number |  |
| 2 | 0 to 4 | Serial number (MSB) |  |
|  | 5 to 7 | Manufacturer code (LSB) | Required. Code obtained from SAE or RVIA. |
| 3 | 0 to 7 | Manufacturer code |  |
| 4 | 0 to 2 | Node instance | For devices implementing multiple RV-C nodes (normally 0) |
|  | 3 to 7 | Function instance | Optional, intended to allow multiple instances of the same RV-C node, normally 0 |
| 5 | 0 to 7 | Compatibility Field | Optional, normally 0 |
| 6 | 0 | Reserved | Always 0 |
|  | 1 to 7 | Compatibility Field | Optional, normally 0 |
| 7 | 0 to 3 | Compatibility Field | Optional, normally 0 |
|  | 4 to 6 | Compatibility Field | Always 0 |
|  | 7 | Arbitrary address capable | Required. 1 – Node supports address claiming 0 – Node uses a fixed source address |

Proposed Changes in RED:

3.3.1 Introduction

The RV-C node that has no assigned SA address may use the node claiming procedure to get an unique SA address. Unless specified as part of a device type’s definition, each node on an RV-C network should claim exactly one source address, even if it implements more than one device type or instance of a device type.

3.3.2 Source Address claiming

The RV-C node requesting dynamically an SA begins with an address request message. Every RV-C node shall be able to respond to the address request message with an address claimed message. RV-C nodes that are "hard wired" to an SA shall respond to the address request in order to prevent a dynamically addressed RV-C node from taking its address. A dynamically addressed RV-C node will try addresses until it finds one unclaimed. Once it finds an unclaimed address, it sends the address claimed message.

Table 3.3.2a defines the address request message.

Table 3.3.2a - Address request message

| Attribute | Value definition |
| --- | --- |
| DGN | EA00h |
| DGN-High | EAh |
| DGN-Low | Address desired |
| Priority: | 6 |
| Data length | 8 bytes |
| Broadcast gap | As needed |
| Source address | 254 (no address has been claimed) |
| Data 0 | 0 |
| Data 1 | 238 |
| Data 2 | 0 |
| NOTE It will be seen that that this is just an ordinary request for DGN EE00h, but directed at a particular address. |  |

Table 3.3.2b defines the address claimed message.

Table 3.3.2b - Address claimed

| Attribute | Value definition |
| --- | --- |
| DGN | EE00h |
| DGN-High | EEh |
| DGN-Low | 00h |
| Piority Priority | 6 |
| Broadcast gap | On request only |
| Source address | Assigned SA |
| Data 0 to 7 | "ADDRESS_CLAIM" (8-byte identifier, see clause 3.3.3) |

In order to reduce the possibility of mis-configured RV-C nodes (ending up with the very same SA), RV-C nodes used in the same network should try to claim different SAs. In most cases, RV-C nodes should start with the address given for one of its RV-C node device types in Table 7.2. The RV-C node shall not attempt to claim an address that might be used by a statically addressed RV-C node. If an address is already claimed, the RV-C node shall try the next address lower, and thus count downwards.

In rare cases a manufacturer may know that many of its nodes for which the same preferred dynamic address range applies will be regularly installed on the same RV-C network. In order to reduce network traffic at startup, the manufacturer may choose to have its nodes begin address claiming at a lower point in the preferred dynamic address range as opposed to the highest value. For instance, if a vendor of RV-C window shades (preferred dynamic address range 128-143) sells a package that implements more than a dozen nodes, each requiring its own source address, they may design their nodes so that the first begins address claiming at 140, the second begins claiming at 139, the third at 138, and so on.

The ADDRESS_CLAIM field as described in Section 3.3.3 is used to determine the priority of RV-C nodes in address claiming. RV-C nodes with a higher priority ADDRESS_CLAIM field may "take" an address from a lower priority RV-C node, though it is acceptable for manufacturers to choose to avoid this practice (for example, they may choose to avoid “taking” the addresses of lower priority nodes except when all other addresses in the preferred range are in use). Statically addressed RV-C nodes may keep their address. Dynamically addressed RV-C nodes shall yield to static RV-C nodes, or to dynamic RV-C nodes with a higher priority ADDRESS_CLAIM field.

An RV-C node with dynamic addresses shall monitor network traffic and if they see another RV-C node using its address and it has a higher priority ADDRESS_CLAIM field it shall automatically run through the procedure and claim a new address.

3.3.3 ADDRESS_CLAIM field

The ADDRESS_CLAIM field is used as a method of uniquely identifying RV-C nodes during the address claiming procedure and for network troubleshooting. When two or more nodes attempt to claim the same source address, the ADDRESS_CLAIM field is used for arbitration. The ADDRESS_CLAIM field should be treated as an eight byte value with the “Arbitrary Address Capable” bit being the most significant bit. This ensures statically addressed nodes will always have the highest priority.

The ADDRESS_CLAIM field is defined in Table 3.3.3.

Table 3.3.3: ADDRESS_CLAIM field

| Byte | Bit | Name | Description |
| --- | --- | --- | --- |
| 0 | 0 to 7 | Serial number (LSB) | Optional. Required if multiple nodes from the same manufacturer may be present on the network. |
| 1 | 0 to 7 | Serial number |  |
| 2 | 0 to 4 | Serial number (MSB) |  |
|  | 5 to 7 | Manufacturer code (LSB) | Required. Code obtained from SAE or RVIA. |
| 3 | 0 to 7 | Manufacturer code |  |
| 4 | 0 to 2 | Node instance - DEPRECATED | Deprecated. For devices implementing multiple RV-C nodes (normally 0) |
|  | 3 to 7 | Function instance - DEPRECATED | Deprecated. Optional, intended to allow multiple instances of the same RV-C node, normally 0 |
| 5 | 0 to 7 | Compatibility Field | Optional, normally 0 |
| 6 | 0 | Reserved | Always 0 |
|  | 1 to 7 | Compatibility Field | Optional, normally 0 |
| 7 | 0 to 3 | Compatibility Field | Optional, normally 0 |
|  | 4 to 6 | Compatibility Field | Always 0 |
|  | 7 | Arbitrary address capable | Required. 1 – Node supports address claiming 0 – Node uses a fixed source address |