New Section 3.3.2.2 -

3.3.2.2 - Persistent Conflict

If a device receives a message from its own source address, it shall immediately transmit ADDRESS_CLAIMED.  If additional messages continue to be received, it shall transmit ADDRESS_CLAIMED every second for as long as messages continue to be received.  After waiting one second to allow the problem to resolve, it shall also broadcast DM_RV every second, with the following characteristics:

- The Red Lamp flag will be ON (01b).

- The SPN shall be the Standard SPN 14 - Node Source Address.

- The FMI shall be 11 - Failure Not Identifiable.

- Other fields as appropriate to the device.

The device shall continue to operate and transmit its operational messages normally.  Note that address-specific messages, such as proprietary messages, cannot be reliably identified when a conflict exists.

If a ADDRESS_CLAIMED message is received, the device shall respond according to the normal address claiming process, but ADDRESS_CLAIMED shall not be transmitted more frequently than once per second.   Optionally, after determining that the conflict is not otherwise resolving, a device may abandon the address and enter the dynamic claiming process, targeting a new address, on its own initiative.

Add to Table 7.3

Standard SPNs

These SPNs apply to devices of all types. The range of 0 to 255 is reserved for this purpose. Node-specific SPNs shall be 256 or greater. Standard SPNs must be encoded as non-instanced failures, per section Error: Reference source not found. In each case, the MSB is zero, and the ISB and LSB contain the SPN value (see Table Error: Reference source not found).

Table Error: Reference source not found — Standard SPNs

| Value | Description |
| --- | --- |
| 0 | Specific Point Unknown |
| 1 | Node Microprocessor (or ECM) |
| 2 | Node Analog-Digital Converter |
| 3 | Node Settings or Configuration |
| 4 | Node RAM |
| 5 | Node Power Supply |
| 6 | Node Temperature |
| 7 | Node Clock / Timer |
| 8 | Node Date / Time. Usually indicates a product that requires a clock DGN from another source. |
| 9 | Node RV-C Connection. Usually indicates a product that requires a specific DGN from another source. |
| 10 | Node Serial/ID Number |
| 11 | Node Processor Supervisor (Watchdog) |
| 12 | Node Firmware |
| 13 | Node Internal Subnetwork Communications. |
| 14 | Connection |
| 15 | Broker |
| 16 | WRV-C Security |
| 17 | Node Source Address |