Please make all addition in red

Justification:

5.3 Standard Physical Units

Table 5.3 defines the basic data scales for different physical units.

Table 5.3 - Basic data scale definition

| Unit | Data type | Min | Max | Precision | Special values |
| --- | --- | --- | --- | --- | --- |
| % | uint8 | 0 | 125 | 0.5% | - |
| Instance | uint8 | 0 | 250 |  | 0 = all 254 – Out of range / not configured 255 – Data not available |
| °C | uint8 | -40 | 210 | 1 °C | - |
|  | uint16 | -273 | 1735 | 0.03125 °C | - |
| Vac or Vdc (volt) | uint8 | 0 | 250 | 1 V | - |
|  | uint16 | 0 | 3212.5 | 0.050 V | - |
| Aac or Adc (amperage) | uint8 | 0 | 250 | 1 A | - |
|  | uint16 | -1600 | 1612.5 | 0.05 A | 0A = 7D00 |
|  | uint32 | –2,000,000.000A | 2,221,081.200A | 0.001 A | 0A = 77359400 |
| Hz (hertz) | uint8 | 0 | 250 | 1 Hz |  |
| W (watts) | uint16 | 0 | 65530 | 1 W |  |
| Amp-Hours | uint16 | 0 | 65530 | 1 A•h |  |
| Ω (ohms resistance) | uint16 | 0 | 65530 | 1 Ω | 0 to 65530 |

Often a device may be called upon to report a value as a percent but the device may only be capable of a few discrete levels. For example, an Air Conditioner may report Fan Speed (a percent value), but have only two (Low and High) or three (Low, Medium, High) possible speeds. In such cases, the following method for reporting is recommended.

First, note that 0 always means Off (or the equivalent). Other values are divided as equally as possible among the possible states. Thus a two-stage fan would consider values from 1-100 (50%) as "Low" and 101-200 (100%) as "High". A three-stage fan would use 1-66 (33%) as "Low", 67-133 (66.5%) as "Medium", and 134-200 (100%) as "High". A five-stage fan would use intervals of 1-40, 41-80, 81-120, 121-160, and 161-200.

When reporting the status, the device should use the highest value in the applicable range. e.g. For a three-stage fan, 0, 66, 133, and 200. When parsing commands, it should accept any value in the range. e.g. If the three-stage fan receives a command to go to 100 (50%), it would consider this "Medium" and report a speed of 133.

This scheme assumes that the steps or levels are evenly distributed along the spectrum from Off to 100% On. If the steps are not reasonably uniform, a more precise representation of the values is recommended.