6.11.2   Chassis Mobility Status

This DG can be queried by any device that needs to know whether the RV is in motion or immobilized. Table 6.11.2a defines the DG attributes, and Table 6.11.2b defines the signal and parameter attributes.

Table 6.11.2a — DG definition
	DG attribute
	Value

	Name
	CHASSIS_MOBILITY_STATUS

	DGN
	1FFF4h

	Default priority
	4

	Maximum broadcast gap
	5000 ms

	Normal broadcast gap
	on change

	Minimum broadcast gap
	500 ms

	Number of frames
	1

	ACK requirements
	None


Table 6.11.2b — Signal and parameter definition
	Byte
	Bit
	Name
	Data Type
	Unit
	Value definition

	0 to 1
	-
	Engine RPMs
	uint16
	rpm
	Precision = 0.125 rpm
Value Range = 0 to 8191.625 rpm 
65534 – Error
65535 - Unknown

	2 to 3
	-
	Vehicle speed
	uint16
	kph
	Precision = 1/256 kph
Value Range = 0 to 255 kph 
65534 - Error
65535 - Unknown

	4
	0 to 1
	Park brake status
	uint2
	-
	00b - Park Brake Released. Coach is free to roll
01b - Park Brake Engaged. Coach is immobilized

	
	2 to 3
	Transmission lock status
	uint2
	-
	00b - Transmission is not locked
01b - Transmission is locked (shall not go into gear)

	
	4 to 5
	Engine lock status
	uint2
	-
	00b - Engine is free to start
01b - Engine is locked

	5
	0 to 1
	Ignition switch status
	uint2
	-
	00b - Off
01b - On

	
	2 to  4  3
	Accessory switch status
	uint2
	-
	00b - Off 
01b - On

	
	4 to 7
	Turn Signal Intent
	uint4
	-
	The state of the driver’s control.  It remains constant while the signal bulbs are flashing.

0000b – Off

0001b – Left Turn

0010b – Right Turn

0011b – Hazard Signal

	6
	
	Transmission Current Gear
	uint8
	-
	124 = Reverse 1
125 = Neutral
126 = Forward 1
127 = Forward 2 etc.
251 = Park (Note: Most heavy transmissions do not have a 'Park', but are parked in Neutral.)

	7
	
	Transmission Gear Selected
	uint8
	-
	Same format as Current Gear


