Justification: It appears that during formatting between Nov-2020 and Apr-2021, information relating to biax operation was removed.  I could not find any amendment requests during that time period.  This amendment restores the removed information and moves the position definitions from the Name to the Value Definition for Clarity.  The Names have been numbered to maintain unique names.  Also, the first field of Byte 2 was misnumbered.

6.13.3 Leveling Control Command

This is the primary DG command for controlling the leveling systems. The Operating Mode determines the nature of the command. When a manual mode is chosen (#4 or #6), the remainder of the DG must be analyzed to determine which jacks or bags are being controlled. In the other modes the rest of the DG is ignored.

There are two general methods for levelers to be addressed. In a conventional four-point system, they are labeled left-front, right-front, left-rear, and right-rear. In a bi-axis system, units move in pairs and are labeled front, rear, left, and right. In a three-point system they are front, left rear, and right rear.

Table 6.13.3a defines the DG attributes, and Table 6.13.3b defines the signal and parameter attributes.

Table 6.13.3a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | LEVELING_CONTROL_COMMAND |
| DGN | 1FFEEh |
| Default priority | 3 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | on change or 100 ms when manually controlling the system |
| Minimum broadcast gap | 100 ms when system active |
| Number of frames | 1 |
| ACK requirements | ACK always, LEVELING_CONTROL_STATUS |

Table 6.13.3b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Operating mode | uint8 | - | 0 – Inactive 1 - Suspension raise (all air springs) active 2 - Suspension dump (all air springs) active 3 - Store active (store jacks/go to ride height) 4 - Manual air leveling (Deprecated in favor of 41-44) 5 - Automatic air leveling started 6 - Start deploy kick-down jacks 7 - Manual hydraulic leveling (Deprecated in favor of 71-74) 8 - Automatic hydraulic/electric leveling started 9 - Kneel started (dump front air springs only) 10 – Auto hitch height 11 - Abort Function 12 – Careen Left. (i.e. Tilt towards street side.) 13 – Careen Right. (i.e. Tilt towards curb side.) 14 – Squat. (i.e. Dump rear bags only.) 34 - Jack calibration mode 41 - Manual air leveling - Four point 42 - Manual air leveling - Biax mode 43 - Manual air leveling - 3 point mode 44 - Manual air leveling - Multi point mode 5+ 51 – Automatic air leveling – Four point 52 – Automatic air leveling – Biax mode 53 - Automatic air leveling – 3 point mode 54 - Automatic air leveling - Multi point mode 5+ 71 - Manual hydraulic/electric leveling - Four point 72 - Manual hydraulic/electric leveling - Biax mode 73 - Manual hydraulic/electric leveling - 3 point mode 74 - Manual hydraulic leveling - Multi point mode 5+ 81 – Automatic hydraulic leveling – Four point 82 – Automatic hydraulic leveling – Biax mode 83 – Automatic hydraulic leveling – 3 point mode 84 - Automatic hydraulic leveling - Multi point mode 5+ 99 - Clear Jack error |
| 1 | 0 to 1 | Jack Retract/Air Spring Dump 1 | uint2 | - | 00b - Off 01b - On Four point mode – Left Rear Biax mode – Rear 3 point mode – Left Rear Multi point mode – Left Rear |
|  | 2 to 3 | Jack Extend/Air Spring Fill 1 | uint2 | - | 00b - Off 01b - On Four point mode – Left Rear Biax mode – Rear 3 point mode – Left Rear Multi point mode – Left Rear |
|  | 4 to 5 | Jack Retract/Air Spring Dump 2 | uint2 | - | 00b - Off 01b - On Four point mode – Right Front Biax mode – Front 3 point mode – N/A Multi point mode – Right Front |
|  | 6 to 7 | Jack Extend/Air Spring Fill 2 | uint2 | - | 00b - Off 01b - On Four point mode – Right Front Biax mode – Front 3 point mode – N/A Multi point mode – Right Front |
| 2 | 0 to 1 | Jack Retract/Air Spring Dump 3 | uint2 | - | 00b - Off 01b - On Four point mode – Right Rear Biax mode – Right 3 point mode – Right Rear Multi point mode – Right Rear |
|  | 2 to 3 | Jack Extend/Air Spring Fill 3 | uint2 | - | 00b - Off 01b - On Four point mode – Right Rear Biax mode – Right 3 point mode – Right Rear Multi point mode – Right Rear |
|  | 4 to 5 | Jack Retract/Air Spring Dump 4 | uint2 | - | 00b - Off 01b - On Four point mode – Left Front Biax mode – Left 3 point mode – Front Multi point mode – Left Front |
|  | 6 to 7 | Jack Extend/Air Spring Fill 4 | uint2 | - | 00b - Off 01b - On Four point mode – Left Front Biax mode – Left 3 point mode – Front Multi point mode – Left Front |
| 3 | 0 to 1 | Jack Retract/Air Spring Dump 5 | uint2 | - | 00b – Off 01b - On Four point mode – N/A Biax mode – N/A 3 point mode – N/A Multi point mode – Left Middle |
|  | 2 to 3 | Jack Extend/Air Spring Fill 5 | uint2 | - | 00b – Off 01b - On Four point mode – N/A Biax mode – N/A 3 point mode – N/A Multi point mode – Left Middle |
|  | 4 to 5 | Jack Retract/Air Spring Dump 6 | uint2 | - | 00b – Off 01b - On Four point mode – N/A Biax mode – N/A 3 point mode – N/A Multi point mode – Right Middle |
|  | 6 to 7 | Jack Extend/Air Spring Fill 6 | uint2 | - | 00b – Off 01b - On Four point mode – N/A Biax mode – N/A 3 point mode – N/A Multi point mode – Right Middle |
| 4 | 0 to 1 | Jack Retract 7 | uint2 | - | 00b – Off 01b - On Four point mode – N/A Biax mode – N/A 3 point mode – N/A Multi point mode – Tongue |
|  | 2 to 3 | Jack Extend 7 | uint2 | - | 00b – Off 01b - On Four point mode – N/A Biax mode – N/A 3 point mode – N/A Multi point mode – Tongue |