# RV-C Specification Submission -

# Furnace

# Proposed Changes in Red:

6.15  Furnace

6.15.1 Introduction

The furnace is a primary source of comfort heat in the RV. Heat elements that are part of an air conditioning unit are supported under Air Conditioner. Table 6.15.1 defines the furnace DGs.

Table 6.15.1 — Furnace definition

| Device attribute | Value |
| --- | --- |
| Category | Comfort systems |
| Default Source Address | 94 to 96 |
| Dynamic Address Range | 192 to 207 |
| Instance | Multiple |

Multiple source addresses are allocated, but furnaces are identified by the Instance in each DGN. These correspond to “zones” in the general terminology. There is no set definition for the location of each zone in a coach.

The furnace generally operates in an “automatic” mode, consulting the thermostat assigned to the same Instance for the necessary temperature and user input data. The mode may be overridden for diagnostic purposes, or as part of a larger climate control scheme.

Systems that use multiple heat exchangers with a single combustion unit should report as multiple instances. Each heat exchanger reports as a “furnace”, with the fan speed set appropriately and the heat output level being that of the central combustion unit. Commands to set the heat output level for any instance should be applied to the central unit.

6.15.2 Furnace Status

This is the general furnace status DGN. Table 6.15.2a defines the DG attributes, and Table 6.15.2b defines the signal and parameter attributes.

Table 6.15.2a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | FURNACE_STATUS |
| DGN | 1FFE4h |
| Default priority | 6 |
| Maximum broadcast gap | 5000 ms |
| Normal broadcast gap | on change |
| Minimum broadcast gap | 500 ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.15.2b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | see Table 5.3 Corresponds to “Zones” in user terminology. |
| 1 | 0 to 1 | Operating mode | uint2 | - | 00b — Automatic 01b — Manual (Furnace shall ignore thermostat information.) |
|  | 2 to 7 | Heat source | uint6 | - | 0 — Combustion 1 — AC power primary 2 — AC power secondary 3 — Engine heat 4 — Hydronic heat – Combustion This furnace instance is a heat exchanger in a hydronic system. The system’s boiler is currently using combustion as its heat source. 5 — Hydronic heat – Electric 6 — Hydronic heat – Gas/electric (both) |
| 2 | - | Circulation fan speed | uint8 | % | see Table 5.3 One-speed fans shall use 0% and 100%. Two-speed fans shall use 0%, 50%, and 100%. |
| 3 | - | Heat output level | uint8 | % | see Table 5.3 |
| 4 | - | Dead band | uint8 | °C | Precision = 0,1 °C Value range = 0,0 to 25,0 °C This is the amount over and under the set point that the furnace will tolerate. A larger value reduces cycling. |
| 5 | - | Second stage dead band | uint8 | °C | Precision = 0,1 °C Value range = 0,0 to 25,0 °C This is the amount over the set point that will trigger a second stage (“high power”), if available on the furnace. |
| 6 | 0 to 1 | ZoneOvercurrent Status | uint2 | - | 00b - No overcurrent detected 01b - Overcurrent detected |
|  | 2 to 3 | ZoneUndercurrent Status | uint2 | - | 00b - No undercurrent detected 01b - Undercurrent detected |
|  | 4 to 5 | ZoneTemperature Status | uint2 | - | 00b - Temperature normal 01b - Temperature warning |
|  | 6 to 7 | ZoneAnalog Input Status | uint2 | - | 00b - Off (Inactive) 01b - On (Active) |

6.15.3 Furnace Command

This DGN allows external control of the furnace. The format is identical to FURNACE_STATUS. An Instance of Zero indicates that the settings should be applied to all furnace instances. Values of 255 indicate that the particular datum should not be changed. Table 6.15.3 defines the DG attributes, and Table 6.15.3b defines the signal and parameter attributes.

Table 6.15.3 — DG definition

| DG attribute | Value |
| --- | --- |
| Name | FURNACE_COMMAND |
| DGN | 1FFE3h |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | NAK, FURNACE_STATUS |

Table 6.15.3b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | see Table 5.3 |
| 1 | 0 to 1 | Operating mode | uint2 | - | see Table 6.15.2b |
|  | 2 to 7 | Heat source | uint6 | - | see Table 6.15.2b |
| 2 | - | Circulation fan speed | uint8 | % | see Table 5.3 see Table 6.15.2b |
| 3 | - | Heat output level | uint8 | % | see Table 5.3 |
| 4 | - | Dead band | uint8 | °C | see Table 6.15.2b |
| 5 | - | Second stage dead band | uint8 | °C | see Table 6.15.2b |

6.15.6 Furnace Configuration Status

This DG describes configuration information for the furnace. Table 6.15.6a defines the DG attributes, and Table 6.15.6b defines the signal and parameter attributes.

Table 6.15.6a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | FURNACE_CONFIGURATION_STATUS |
| DGN | 1FDA1 |
| Default priority | 6 |
| Maximum broadcast gap | None |
| Normal broadcast gap | On change or request. |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.15.6b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | See Table 5.3 Corresponds to “Zones” in user terminology. |
| 1 | 0 to 3 | Temperature sensor location | uint4 | - | Location of temperature sensor that furnace will use when in Automatic Operating mode (see “Operating mode” field of FURNACE_STATUS) and not relying on temperature information from the RV-C network. 0 – Furnace to make use of external temperature sensor 1 – Furnace to make use of integrated temperature sensor |

6.15.7 Furnace Configuration Command

This DGN allows changes in furnace configuration. Table 6.15.7a defines the DG attributes. The signal and parameter attributes have the same format as FURNACE_CONFIGURATION_STATUS (see Table 6.15.6b).

Table 6.15.7a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | FURNACE_CONFIGURATION _COMMAND |
| DGN | 1FDA0 |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| ACK requirements | None |