Justification:

Correct typo in test case 88A-S-01 (fan speed of ‘0’ while in “manual” mode does not mean to set the fan speed automatically).

Original Section:

6.16.14.4 Profile 88A: Active Thermostat w/ Internal Temperature Sensor

This profile applies to thermostats that actively manage a networked air conditioner and which have an internal temperature sensor. The air conditioner(s) is required to be compliant with Profile 103H (Slave Air Conditioner with Hysteresis).

Prerequisite: 88B Base Thermostat, 88Z Temperature Sensor

Reporting

| ID | Datum | Test | Required Response |
| --- | --- | --- | --- |
| 88A-S-01 | THERMOSTAT_COMMAND_1 - OpMode Cool/Auto/Off | 1. THERMOSTAT_COMMAND_1 is sent with Operating Mode = 1 (Cool), Set Point Cool set 5 degF below the current temperature, Fan Speed 0 (Automatic). 2. After step 1 is complete, the test continues with Operating Mode = 0 (Off). 3. As step 1, but with Operating Mode = 3 (Auto) 4. After step 3 is complete, the test continues with Operating Mode = 0 (Off). | 1. AIR_CONDITIONER_COMMAND is sent immediately with Operating Mode 1 (Manual), Output Level 200 (100%), Fan Speed 0 (Automatic). 2. AIR_CONDITIONER_COMMAND is sent immediately with Operating Mode 1 (Manual), Output Level 0 (0%). 3. Same as Step 1 4. Same as Step 2. |

Proposed Changes in RED:

6.16.14.4 Profile 88A: Active Thermostat w/ Internal Temperature Sensor

This profile applies to thermostats that actively manage a networked air conditioner and which have an internal temperature sensor. The air conditioner(s) is required to be compliant with Profile 103H (Slave Air Conditioner with Hysteresis).

Prerequisite: 88B Base Thermostat, 88Z Temperature Sensor

Reporting

| ID | Datum | Test | Required Response |
| --- | --- | --- | --- |
| 88A-S-01 | THERMOSTAT_COMMAND_1 - OpMode Cool/Auto/Off | 1. THERMOSTAT_COMMAND_1 is sent with Operating Mode = 1 (Cool), Set Point Cool set 5 degF below the current temperature, Fan Speed 0 (Automatic). 2. After step 1 is complete, the test continues with Operating Mode = 0 (Off). 3. As step 1, but with Operating Mode = 3 (Auto) 4. After step 3 is complete, the test continues with Operating Mode = 0 (Off). | 1. AIR_CONDITIONER_COMMAND is sent immediately with Operating Mode 1 (Manual), Output Level 200 (100%), Fan Speed 0 (Automatic) and fan speed set automatically by the thermostat. 2. AIR_CONDITIONER_COMMAND is sent immediately with Operating Mode 1 (Manual), Output Level 0 (0%). 3. Same as Step 1 4. Same as Step 2. |