Justification: The Thermostat Status 2 already defines a reduced noise mode. We want to extend this selection with two additional modes: an eco-mode (for low energy operation) and a turbo-mode (for high-performance operation). They behave like the reduce noise mode as supplementary modes to e.g. heating or cooling.

Changes highlighted in red.

# 6.16.3 Thermostat Status 2

This communicates user intentions. Table 6.16.3a defines the DG attributes, and Table 6.16.3b defines the signal and parameter attributes.

Table 6.16.3a – DG definition

| DG Attribute | Value |
| --- | --- |
| Name | THERMOSTAT_STATUS_2 |
| DGN | 1FEFAh |
| Default Priority | 6 |
| Maximum broadcast gap | 5000ms |
| Normal broadcast gap | On change |
| Minimum broadcast gap | 500ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.16.3b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | see Table 5.3 A value of 0 indicates that the fan speed shall be set automatically according to the demand for cooling. A non-zero value indicates that the speed shall be manually overridden. |
| 1 | - | Current schedule instance | uint8 | - | see Table 5.3 Indicates which schedule instance is currently active. |
| 2 | - | Number of schedule instance | uint8 | - | Indicates total capacity for schedule instances. |
| 3 | 0-1 | Reduced noise mode | bit | - | Indicates that the units shall operate in a low noise emission mode 00b – Disabled 01b – Enabled |
| 3 | 2-3 | Eco mode | bit | - | Indicates that the units shall operate in a low energy operation mode 00b – Disabled 01b – Enabled |
| 3 | 4-5 | Turbo mode | bit | - | Indicates that the units shall operate in a high-performance mode 00b – Disabled 01b – Enabled |