Justification: The Thermostat Command 2 already defines a reduced noise mode. We want to extend this selection with two additional modes: an eco-mode (for low energy operation) and a turbo-mode (for high-performance operation). They behave like the reduce noise mode as supplementary modes to e.g. heating or cooling.

Changes highlighted in red.

# 6.16.5 Thermostat Command 2

This DGN provides several methods to manually adjust the set point. If Schedule Mode is Enabled while the setpoint is manually adjusted, the setpoint will change with the next scheduled change. If Schedule Mode is Disabled, then the setpoint will not change. Some thermostats may automatically revert to the original setpoint after a period of time. This behavior is not specifically addressed in this DGN.

To put the thermostat in a “Storage” mode, this DGN should be used in conjunction with THERMOSTAT_COMMAND_1 and the Schedule Mode should be set to Disabled. To force the thermostat to resume the scheduled mode the Current Schedule Instance should be set to 251. For example, when bringing the thermostat out of Storage (Instance 250), the Instance should be set to 251. The thermostat should check its schedule and choose the appropriate Instance (e.g. “Wake”) accordingly. Table 6.16.5a defines the DG attributes, and Table 6.16.5b defines the signal and parameter attributes.

Table 6.16.5a — DG definition

| DG Attribute | Value |
| --- | --- |
| Name | THERMOSTAT_COMMAND_2 |
| DGN | 1FEF8h |
| Default Priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | NAK, THERMOSTAT_STATUS_2 |

Table 6.16.5b— Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | see Table 5.3 Corresponds to “Zones” in user terminology. |
| 1 | - | Current schedule instance | uint8 | - | see Table 5.3 251 — Reset to “current” instance. Shall Force the unit into the indicated mode. |
| 2 | 0-1 | Reduced noise mode | bit | - | Indicates that the units shall operate in a low noise emission mode 00b – Disabled 01b – Enabled |
| 2 | 2-3 | Eco mode | bit | - | Indicates that the units shall operate in a low energy operation mode 00b – Disabled 01b – Enabled |
| 2 | 4-5 | Turbo mode | bit | - | Indicates that the units shall operate in a high-performance mode 00b – Disabled 01b – Enabled |