Justification: The Thermostat Schedule Status 1 defines a “Start hour” byte that allows a schedule based on the real time in local time. We are developing a low-energy climate device that has no real time clock available. We therefore propose an additional definition of relative time within the same byte. This allows devices without a real time clock to specify a schedule relative to the current time, e.g. to perform an action in 2 hours.

Changes highlighted in red.

# 6.16.7 Thermostat Schedule Status 1

Table 6.16.7a defines the DG attributes, and Table 6.16.7b defines the signal and parameter attributes.

Table 6.16.7a — DG definition

| DG Attribute | Value |
| --- | --- |
| Name | THERMOSTAT_SCHEDULE_STATUS_1 |
| DGN | 1FEF7h |
| Default Priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | On change |
| Minimum broadcast gap | 500ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.16.7b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | see Table 5.3 Corresponds to “Zones” in user terminology. |
| 1 | - | Schedule mode instance | uint8 | - | 0 — “Sleep” 1 — “Wake” 2 — “Away” 3 — “Return” 4 to 249 — Additional Instances 250 — Storage |
| 2 | - | Start hour | uint8 | h | For real time: Precision = 1 h Value range = 0 to 23 0 - 12:00 AM 12 – 12:00 Noon 23 – 11:00 PM This shall be in Local Time For relative time: Precision = 1 h Value range = 24 to 224 24 – 0 hours 25 – 1 hour 152 – 128 hours 224 – 200 hours |
| 3 | - | Start minute | unit8 | min | Precision = 1 min Value range = 0 to 59 |
| 4 to 5 | - | Setpoint temp – Heat | uint16 | Deg C | see Table 5.3 |
| 6 to 7 | - | Setpoint temp – Cool | uint16 | Deg C | see Table 5.3 |

Values for the parameter “Start hour” from 24 to 224 indicate the schedule is based on the time relative to now, rather than being based on the real time. This supports applications where no real time clock is available. The parameter “Start minute” can be used to set minutes for both, relative time and real time.

Examples:

If from now the unit should execute an action in 14 hours, the parameter “Start hour” should be set to 38d.

If from now the unit should execute an action in 8 hours and 25 minutes, the parameter “Start hour” should be 32d and the parameter “Start minute” should be 25d.

If from now the unit should execute an action in 30 minutes, the parameter “Start hour” should be 24d (equals 0 hours) and “Start minute” should be 30d.

# 6.16.9 Thermostat Schedule Command 1

There are two DGNs defined for thermostat schedule command. Generally, changing the set points for the currently active schedule instance will not change the actual set points active at the moment. To trigger that change, a THERMOSTAT_COMMAND_2 should be sent immediately after the change, with Current Schedule Instance set appropriately. Table 6.16.9 defines the DGN attributes. The format for the signal and parameter attributes is identical to THERMOSTAT_SCHEDULE_STATUS_1 (see Table 6.16.7b).

Table 6.16.9 — DG definition

| DG Attribute | Value |
| --- | --- |
| Name | THERMOSTAT_SCHEDULE_COMMAND_1 |
| DGN | 1FEF5h |
| Default Priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | NAK, THERMOSTAT_SCHEDULE_STATUS_1 |