Please include a Justification and make any changes in Red

6.17.10Service Points

The SPNs follow the general method for multi-instance products. Faults are reported with the Instance in the Intermediate Byte (ISB) and a non-zero value in the Most Significant Byte (MSB). These are the allowable Service Points for this DGN (see Table 6.17.10).

Table 6.17.10 — Service Points

| MSB | ISB | LSb | Description |
| --- | --- | --- | --- |
| 0 | 0-FFh | 0-7 | Node-Global Service Points. Combine the ISB and LSb to calculate. Values from 0-FFh are Standard SPNs – see Table 7.3 |
| 1 | Instance | 0 | Fan |
| 1 | Instance | 1 | Compressor |
| 1 | Instance | 2 | Heat Source |
| 1 | Instance | 3 | Temperature |
| 1 | Instance | 4 | Outside Air Sensor |
| 1 | Instance | 5 | Coil Sensor |
| 1 | Instance | 6 | No AC Available |
| 1 | Instance | 7 | Defrosting |
| 2 | Instance | 0 | Subnet Communication |
| 2 | Instance | 1 | Heat Configuration |
| 2 | Instance | 2 | Dehumidifier Configuration |
| 2 | Instance | 3 | Internal Temperature Sensor |
| 2 | Instance | 4 | Pressure Switch |
| 2 | Instance | 5 | Air Flap |
| 2 | Instance | 6 | Outdoor Coil |
| 2 | Instance | 7 | Defrost Lockout |
| 3 | Instance | 0 | DC Voltage |
| 3 | Instance | 1 | Outdoor Coil Sensor |
| 3 | Instance | 2 | Compressor Inlet Sensor |
| 3 | Instance | 3 | Compressor Outlet Sensor |
| 3 | Instance | 4 | Coil Sensor #2 |