Justification:

Adding a command to save the current settings loaded onto the device as OEM-specific settings. Pairs with ‘Reset to OEM-Specific Settings’, which can reset the settings to the saved settings via the new command.

Also modifying the 'Value Definition' column for 'Reset to OEM-Specific Settings' because the original was replicating the 'Reset to default settings' command. Clarification now clearly states that command 'Reset to OEM-Specifc Settings' resets settings to OEM saved setting values.

6.2.1 General Purpose Reset

A general purpose reset supplies a method of resetting a device on the network. Like the Proprietary DG, it is directed at a specific source address. Nodes are not required to support this DGN.

If Reset to Default Settings is indicated, the node should also respond with the appropriate configuration status DGNs. Table 6.2.1a defines the DG attributes, and Table 6.2.1b defines the signal and parameter attributes.

Table 6.2.1a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | GENERAL_RESET |
| DGN | 17F00h DGN-Low - Destination Address |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | ACK Always |

Table 6.2.1b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 to 1 | Reboot | uint2 | - | 00b - No action 01b - Reboot |
|  | 2 to 3 | Clear faults | uint2 | - | 00b - No action 01b - Clear faults |
|  | 4 to 5 | Reset to default settings | uint2 | - | 00b - No action 01b - Restore settings to default values |
|  | 6 to 7 | Reset Statistics | uint2 | - | 00b - No action 01b – Reset Communication Status Statistics (see 6.6) |
| 1 | 0 to 1 | Test Mode | uint2 | - | 00b – Quit testing node 01b – Initiate testing node |
|  | 2 to 3 | Reset to OEM-Specific Settings | uint2 | - | 00b - No action 01b - Restore settings to OEM saved setting values |
|  | 4 to 5 | Reboot/Enter Bootloader Mode | uint2 | - | 00b - No action 01b - Reboot or enter bootloader/programming mode. |
|  | 6 to 7 | Save OEM-Specific Settings | uint2 | - | 00b – No Action 01b – Save currently loaded setting values as OEM-specific settings |