RV-C Submission: Addition of “Generic Revision Status.”

Justification: allow the reporting of device revision information.

6.2 General Purpose Data Groups

6.2.1 to 6.2.5 Not Shown in this Submission

…

6.2.6 Generic Revision Status

This data group provides a generic method for devices to share hardware or firmware revision information. This may be used by an RV manufacturer to ensure that all appliances on a coach are up-to-date before shipping, or by a field technician to see what version of a device they are troubleshooting. Nodes are not required to support this DGN.

Revision information is reported by means of a five byte “Generic Revision Number”, each byte of which is defined in the DG signal and parameter attributes below. See Table 6.2.6d for examples.

This DG is not subject to “Request for DGN” (see 3.2.4.3); instead, GENERIC_REVISION_ REQUEST (below) should be used to request this DG.

Table 6.2.6a defines the DG attributes, and Table 6.2.6b defines the signal and parameter attributes.

Table 6.2.6a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | GENERIC_REVISION_STATUS |
| DGN | 1FD97h |
| Default priority | 7 |
| Maximum broadcast gap | 250ms |
| Normal broadcast gap | N/A |
| Minimum broadcast gap | 25ms |
| ACK requirements | None |

Table 6.2.6b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | DSA | uint8 | - | See Table 7.2. 0-63 – Invalid |
| 1 | - | Instance | uint8 | - | FF for non-instanced devices |
| 2 | 0-4 | Subsystem Index | uint5 | - | The subsystem of the device on which revision information is being reported. 0 – Full system Should not be used if a defined subsystem (see below) is also being reported on for this revision type. 1-24 – Defined per DSA. See table 6.2.6c. 25-29 – Vendor defined. 30 – Error |
|  | 5-7 | Revision Type | uint3 | - | The type of revision information being reported. 0 – Hardware 1 – Firmware 2 – Configuration 3-5 – Open for future development 6 – Error |
| 3 | - | “Generic Revision Number” First Byte - Release Year | uint8 | Year | Precision = 1 year Offset = 2000 AD Value range = 2000 to 2250 The year that this revision was released. See notes below. |
| 4 | - | “Generic Revision Number” Second Byte - Release Month | uint8 | Month | 1 – January, 2 – February , . . . , 12 – December The month that this revision was released. |
| 5 | - | “Generic Revision Number” Third Byte - Version | uint8 | - | 254 – Error Device manufacturers should begin at 1, and increment 1 for every version of this firmware released this month. See notes below. |
| 6-7 | - | “Generic Revision Number” Fourth and Fifth Bytes – Product Series | uint8 | - | The vendor-defined product series that this device belongs to. MSB in byte 6, LSB in byte 7. See notes below. If a vendor only has one product series of a device type, this value should be 1 (that is, MSB:0, LSB:1) |

Note that the first and second bytes of the “Generic Revision Number” (Table 6.2.6b bytes 3-4) contain the year and month that this revision was released - not the year and month that the reporting device was manufactured. For example, this may be the year and month that the writing of this firmware version was completed, or the year and month that this hardware version was first put into production.

The third byte of the “Generic Revision Number” (Table 6.2.6b byte 5) contains the revision ‘version.’ Note that it is uncommon for vendors to release more than one version of firmware for a particular product series within a single month, so generally this number should stay as 1.

The fourth and fifth bytes of the “Generic Revision Number” (Table 6.2.6b bytes 6-7) contain the ‘product series.’ If a vendor manufactures multiple series of an RV-C enabled product, this indicates which series this device belongs to. For example, if an inverter vendor sells multiple different RV-C enabled inverters, each with different firmware, this value might be 1 (MSB:0, LSB:1) for the first, 2 (MSB:0, LSB:2) for the second, and so on. It is up to each vendor to decide how to number their product series. However, if a vendor only has one product series of a device type, this value should be 1.

Table 6.2.6c — Subsystems per DSA

| DSA (See Table 7.2) | Product | Defined Subsystems |
| --- | --- | --- |
| 64 | Generator | None currently defined |
| 65 | Genstart Controller | None currently defined |
| 66,67 | Inverter #1-2 | None currently defined |
| 68 | Control Panel* | None currently defined |
| 69 | Battery State of Charge Monitor | None currently defined |
| 70 | Battery | None currently defined |
| 71 | Chassis Battery S.O.C. | None currently defined |
| 72 | Water/Waste Tank System | None currently defined |
| 73 | LPG Tank System | None currently defined |
| 74 | Converter #1 | None currently defined |
| 75 | Converter #2 | None currently defined |
| 76 | Charge Controller | None currently defined |
| 77 | AC Load Monitor/Controller | None currently defined |
| 78 | AC Fault Protection System | None currently defined |
| 79 | Transfer Switch | None currently defined |
| 80 | Weather Station | None currently defined |
| 81 | Hydraulic/Electric Leveling System Controller | None currently defined |
| 83 | Air Leveling System | None currently defined |
| 84-87 | Slide Room #1-4 | None currently defined |
| 88 | Main Thermostat | None currently defined |
| 89 | Bedroom Thermostat | None currently defined |
| 90-93 | Thermostat #3-6 | None currently defined |
| 94 | Main Furnace (Conventional) | None currently defined |
| #2-3 | Conventional Furnace | None currently defined |
| 97-99 | Aux. Heat (e.g Tile Heat) #1-3 | None currently defined |
| 100 | Furnace (Hydronic) | 1 – Main system controller 2 – Boiler/heater 3 – Auxiliary display panel |
| 101,102 | Water Heater #1-2 | None currently defined |
| 103-106 | Air Conditioners #1-4 | None currently defined |
| 107 | Refrigerator | None currently defined |
| 108 | Aux. Refrigerator (Wine Cooler) | None currently defined |
| 109 | Aux. Freezer | None currently defined |
| 110 | Ice Maker | None currently defined |
| 111 | Stove | None currently defined |
| 112-114 | Audio Entertainment #1-3 | None currently defined |
| 115-117 | Video Entertainment #1-3 | None currently defined |
| 118,119 | TV Lift (Living, Bedroom) | None currently defined |
| 120-125 | Gas Detectors | None currently defined |
| 126 | Active Air Suspension | None currently defined |
| 127 | Water Pump | None currently defined |
| 128 | Tank Autofill | None currently defined |
| 129 | Waste Dump | None currently defined |
| 130 | Awning | None currently defined |
| 131 | DC Dimmer | None currently defined |
| 132 | DC Input, Keypad | None currently defined |
| 133 | Tire Monitor | None currently defined |
| 134 | Window Shade Control | None currently defined |
| 135 | Door Control | None currently defined |
| 136 | GPS | None currently defined |
| 137 | AC Load | None currently defined |
| 138 | DC Motor Controller | None currently defined |
| 139 | DC Disconnect | None currently defined |
| 140 | Generic AC Source | None currently defined |
| 141 | Solar Charge Controller | None currently defined |
| 142 | Roof Fan | None currently defined |
| 143 | External Interface | None currently defined |
| 144 | Generic Alarm | None currently defined |
| 145 | Window Control | None currently defined |
| 146 | DC Load | None currently defined |
| 147 | Step Control | None currently defined |
| 148 | Generic Plumbing Valve | None currently defined |
| 149 | Breaker Panel | None currently defined |
| 150 | Vehicle Seat | None currently defined |
| 249 | Service Tool | None currently defined |
| 250 | System Clock | None currently defined |
| 251 | Data Logger | None currently defined |
| 252 | Chassis Bridge | None currently defined |
| 253 | Network Bridge | None currently defined |

Note that most device manufacturers have in-house names for firmware/hardware revisions that may not adhere to the formatting outlined in Table 6.2.6b. In this case it is highly recommended that device manufacturers maintain documentation connecting the in-house name of each revision with its corresponding RV-C Generic Revision Number. Table 6.2.6d provides an example of such documentation.

Table 6.2.6d — EXAMPLE Revision Documentation

| Vendor Firmware Name | RV-C Generic Revision No. | Notes |
| --- | --- | --- |
| SW036C029 | 24.7.1.0.1 | Initial release |
| SW049D013 | 24.12.1.0.1 | Added support for DGN 1FDACh |
| SY072F065 | 24.12.2.0.1 | Bug fixes |
| SY072G068 | 25.1.1.0.1 | Additional bug fixes |

6.2.7 Generic Revision Request

This DG allows the request of hardware, firmware, or configuration revision information via GENERIC_REVISION_STATUS. This DG is to be used in place of “Request for DGN” (3.2.4.3).

Table 6.2.7a defines the DG attributes, and Table 6.2.7b defines the signal and parameter attributes.

Table 6.2.7a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | GENERIC_REVISION_REQUEST |
| DGN | 1FD96h |
| Default priority | 6 |
| Maximum broadcast gap | None |
| Normal broadcast gap | N/A |
| Minimum broadcast gap | 50ms |
| ACK requirements | GENERIC_REVISION_STATUS |

Table 6.2.7b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | DSA | uint8 | - | See Table 7.2. 0-63 – Invalid |
| 1 | - | Instance | uint8 | - | FF for non-instanced devices |
| 2 | 0-4 | Subsystem Index | uint5 | - | See Table 6.2.6b. 31 – Send all. |
|  | 5-7 | Revision Type | uint3 | - | See Table 6.2.6b. 7 – Send all. |
| 3 | - | Source Address | uint8 | - | 1-253 – Desired source address FFh – Any |