JUSTIFICATION:

Typo in section 6.20.19.

6.20.16 Charger Configuration Status 3

This DGN supplements CHARGER_CONFIGURATION_STATUS. Table 6.20.16a defines the DG attributes and Table 6.20.16b defines the signal and parameter attributes.

Table 6.20.16a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | CHARGER_CONFIGURATION_STATUS_3 |
| DGN | 1FECCh |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | on change |
| Minimum broadcast gap | 50 ms, see 3.2.4.2 |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.20.16b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 0 = Invalid 1 to 250 - Charger Instance |
| 1 to 2 | - | Bulk Voltage | uint16 | V | see Table 5.3 |
| 3 to 4 | - | Absorption Voltage | uint16 | V | see Table 5.3 |
| 5 to 6 | - | Float Voltage | uint16 | V | see Table 5.3 |
| 7 | - | Temperature Compensation Constant | uint8 | mV/K | 0 – 250 mV/K Magnitude of charging voltage adjustment due to temperature |

6.20.17 Charger Configuration Command 3

This changes the configuration information for the Charger. Table 6.20.17a defines the DG attributes. The signal and parameter attributes have the same format as CHARGER_CONFIGURATION_STATUS_3 (see Table 6.20.16b).

Table 6.20.17a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | CHARGER_CONFIGURATION_COMMAND_3 |
| DGN | 1FECBh |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | NAK, CHARGER_CONFIGURATION_STATUS_3 |

6.20.18 Charger Configuration Status 4

This DGN supplements CHARGER_CONFIGURATION_STATUS. Table 6.20.18a defines the DG attributes and Table 6.20.18b defines the signal and parameter attributes.

Table 6.20.18a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | CHARGER_CONFIGURATION_STATUS_4 |
| DGN | 1FEBFh |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | on change |
| Minimum broadcast gap | 50 ms, see 3.2.4.2 |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.20.18b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 0 = Invalid 1 to 250 - Charger Instance |
| 1 to 2 | - | Bulk Time | uint16 | min | Precision = 1 minute Value range = 0 to 65530 minutes |
| 3 to 4 | - | Absorption Time | uint16 | min | Precision = 1 minute Value range = 0 to 65530 minutes |
| 5 to 6 | - | Float Time | uint16 | min | Precision = 1 minute Value range = 0 to 65530 minutes |

6.20.19 Charger Configuration Command 4

This changes the configuration information for the Charger. Table 6.20.19 defines the DG attributes. The signal and parameter attributes have the same format as CHARGER_CONFIGURATION_STATUS_3 (see Table 6.20.19b).

Table 6.20.19 — DG definition

| DG attribute | Value |
| --- | --- |
| Name | CHARGER_CONFIGURATION_COMMAND_4 |
| DGN | 1FEBEh |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | NAK, CHARGER_CONFIGURATION_STATUS_4 |

6.20.20 Charger Configuration Status 5 & Charger Configuration Command 5

(Charger Configuration Status 5 is deprecated and replaced with the DC_SOURCE_CONNECTION_STATUS and DC_SOURCE_CONFIGURATION_COMMAND_3)