Justification:

Purpose of submission is to add new alarm support for the Charger DGN section.

A charger may report an alarm when it is over temperature but may also have functionality to shutdown/disable its operation under temperature, this should be included.

Blue – Opposite condition

Red – New condition

6.20.29 Alarms

Table 6.20.27 lists the alarms.

Table 6.20.29 — Alarms

| Alarm Instance | Description |
| --- | --- |
| 1 | History cleared |
| 2 | Source (AC/DC) power applied |
| 3 | Source (AC/DC) power disabled |
| 16 | Derating Ending |
| 17 | Derate Starting – High Internal Temperature |
| 18 | Derate Starting – High Battery Temperature |
| 19 | Derate Starting – Battery Voltage |
| 20 | Derate Starting – AC Input Voltage |

| 21 | Derate Starting – AC Input Current |
| --- | --- |
| 100 | Charger enabled |
| 101 | Charger disabled |
| 102 | Charger over temperature |
| 103 | Charger under temperature |
| 110 | Transition to bulk stage |
| 111 | Transition to absorption stage |
| 112 | Transition to Overcharge State |
| 113 | Transition to Equalize State |
| 114 | Transition to float stage |
| 115 | Transition to CC/CV Stage |
| 116 | Transition to maintenance stage |
| 120 | Low battery voltage limit |
| 121 | High battery voltage limit |
| 122 | Battery over temperature |
| 123 | Battery under temperature |
| 124 | Battery Disconnected |