Please make all changes in RED and include a justification.

Justification:

6.21  	Generic AC Source

6.21.1 AC Output Introduction

When measuring AC power from a Generator, Inverter, or Transfer Switch, specific DGNs are defined. AC from other sources, such as the Shore Cord, is reported as a Generic AC Source. The following formats apply (see Table 6.21.1a). When an inline device is used in series as a Generic Source for AC Fault Protection then use the specific AC Fault Protection System DSA of 78d for that device.

Table 6.21.1a — Generic AC Source definition

| Device attribute | Value |
| --- | --- |
| Category | Power components |
| Default Source Address | 140 Generic 78 AC Fault Protection System (GMI) |
| Dynamic Address Range | 128 to 143 |
| Instance | Multi-Instance |

AC output is reported using the standard AC Point formats (see Section 6.1). The Instance field is defined in Table 6.21.1b. Note that each of these DGNs may have to be transmitted multiple times to provide information on each line.

Table 6.21.1b — Instance field definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 to 3 | Instance | uint4 | - | 1 — Shore Power |
|  | 4 to 7 | Line | uint4 | - | 1 — Line 1 2 — Line 2 |

6.21.2 Generic AC Output Status 1

Table 6.21.2a defines the DG attributes and Table 6.21.1b defines the instance.

Signal and Parameter definition: See “AC Point” AC_STATUS_1

Table 6.21.2a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | GENERIC_AC_STATUS_1 |
| DGN | 1FEBBh |
| Default priority | 6 |
| Maximum broadcast gap | None |
| Normal broadcast gap | 500 ms when AC active |
| Minimum broadcast gap | 100 ms |
| Number of frames | 1 |
| ACK requirements | None |

6.21.3 AC Output Status 2

Table 6.21.3a defines the DG attributes and Table 6.21.1b defines the instance.

Signal and parameter definition: See “AC Point” AC_STATUS_2

Table 6.21.3a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | GENERIC_AC_STATUS_2 |
| DGN | 1FEBAh |
| Default priority | 6 |
| Maximum broadcast gap | None |
| Normal broadcast gap | 500 ms when AC active |
| Minimum broadcast gap | 100 ms |
| Number of frames | 1 |
| ACK requirements | None |

6.21.4 AC Output Status 3

Table 6.21.4a defines the DG attributes and Table 6.21.4b defines the instance.

Signal and parameter definition: “AC Point” AC_STATUS_3

Table 6.21.4a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | GENERIC_AC_STATUS_3 |
| DGN | 1FEB9h |
| Default priority | 6 |
| Maximum broadcast gap | None |
| Normal broadcast gap | 500 ms when AC active |
| Minimum broadcast gap | 100 ms |
| Number of frames | 1 |
| ACK requirements | None |

6.21.5 AC Output Status 4

Table 6.21.5a defines the DG attributes, and Table 6.21.1b defines the instance.

Signal and parameter definition: “AC Point” AC_STATUS_4

Table 6.21.5a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | GENERIC_AC_STATUS_4 |
| DGN | 1FEB8h |
| Default priority | 6 |
| Maximum broadcast gap | None |
| Normal broadcast gap | 500 ms when AC active |
| Minimum broadcast gap | 100 ms |
| Number of frames | 1 |
| ACK requirements | None |

6.21.5b AC Output Status 5

Table 6.21.5b defines the DG attributes, and Table 6.21.1b defines the instance.

Signal and parameter definition: “AC Point” AC_STATUS_5

Table 6.21.5b — DG definition

| DG attribute | Value |
| --- | --- |
| Name | GENERIC_AC_STATUS_5 |
| DGN | xxxxxh |
| Default priority | 6 |
| Maximum broadcast gap | None |
| Normal broadcast gap | 500 ms when AC active |
| Minimum broadcast gap | 100 ms |
| Number of frames | 1 |
| ACK requirements | None |

6.21.6 AC Fault Configuration Status and Command

These DGNs follow the format of AC Point: ACFAULT_CONFIGURATION_STATUS_1, ACFAULT_CONFIGURATION_STATUS_2, ACFAULT_CONFIGURATION_COMMAND_1, and ACFAULT_CONFIGURATION_COMMAND_2. Instances are defined as above.

The status DGNs are broadcast on request. The command DGNs should be acknowledged with a NAK if necessary and the corresponding status DGN.

| DGN | Hex | Decimal |
| --- | --- | --- |
| GENERIC_ACFAULT_CONFIGURATION_STATUS_1 | 1FEB7h | 130743 |
| GENERIC_ACFAULT_CONFIGURATION_STATUS_2 | 1FEB6h | 130742 |
| GENERIC_ACFAULT_CONFIGURATION_COMMAND_1 | 1FEB5h | 130741 |
| GENERIC_ACFAULT_CONFIGURATION_COMMAND_2 | 1FEB4h | 130740 |

6.21.7 Alarms

Table 6.21.7 lists the alarms.

Table 6.21.7 — Alarms

| Alarm Instance | Description |
| --- | --- |
| 1 | AC Source Switch Event |
| 2 | Impending AC Source Switch |
| 3 | AC Source Detected |
| 4 | AC Source Fault |

6.21.8 Test Profiles

The Generic AC Source profiles are directly inherited from the AC Point profiles, with the only exception being that the instance field shall follow the Instance Field Definition of Table 6.21.1b.

6.21.8.1 Profile 75A: Basic generic AC source

Prerequisites: None

Reporting:

| ID | Datum | AC Point Reference Test ID |
| --- | --- | --- |
| 75A-S-01 | GENERIC_AC_STATUS_1 | 02B-S-01 (see 6.1.10.1) |

6.21.8.2 Profile 75M: Phase Detection Support

Reporting:

| ID | Datum | AC Point Reference Test ID |
| --- | --- | --- |
| 75M-S-01 | GENERIC_AC_STATUS_2 | 02M-S-01 (see 6.1.10.2) |
| 75M-S-02 | GENERIC_AC_STATUS_3 | 02M-S-02 (see 6.1.10.2) |

6.21.8.3 Profile 75C: Fault Control Support

Reporting:

| ID | Datum | AC Point Reference Test ID |
| --- | --- | --- |
| 75C-S-01 | GENERIC_AC_STATUS_4 | 02C-S-01 (see 6.1.10.3) |

Command Response:

| ID | Datum | AC Point Reference Test ID |
| --- | --- | --- |
| 75C-C-01 | GENERIC_ACFAULT_CONFIGURATION_COMMAND_1 | 02C-C-01 (see 6.1.10.3) |
| 75C-C-02 | GENERIC_ACFAULT_CONFIGURATION_COMMAND_2 | 02C-C-02 (see 6.1.10.3) |
| 75C-C-03 | GENERIC_ACFAULT_CONFIGURATION_STATUS_1 | 02C-C-03 (see 6.1.10.3) |
| 75C-C-04 | GENERIC_ACFAULT_CONFIGURATION_STATUS_2 | 02C-C-04 (see 6.1.10.3) |