Please make all addition in red

Justification:

6.32 Transfer Switch

6.32.1 Introduction

A transfer is an electrical switch that controls multiple AC input sources to a single AC output - typically two legs which may or may not be in phase.

RV-C supports up to six transfer switches, each with up to six inputs and one output of one or two legs. If additional output points are needed, then each should be considered a separate transfer switch. If only one transfer switch is installed, it should use a default Instance value of 1. The following formats apply (see Table 6.32.1).

Table 6.32.1 — Transfer switch definition

| Device attribute | Value |
| --- | --- |
| Category | Power components |
| Default Source Address | 79 |
| Dynamic Address Range | 128 to 143 |
| Instance | Multiple |

6.32.2 ATS AC Status

These DGNs broadcast the state of the AC power in and out of the transfer switch. One set of DGNs should be sent for each line and leg. Of course, not all switches will support all DGNs - only the supported DGNs should be broadcast.

The active input line should be broadcast every 500 ms. The inactive inputs should only be broadcast every 5000 ms, or when the DGN is requested.

Table 6.32.2a defines the DG attributes and Table 6.32.2b defines the Byte[0] - Transfer Switch Instance field. The remaining fields for the four DGNs follow the format of the four AC Point Status DGNs. See tables 6.1.2b, 6.1.3b, 6.1.4b, and 6.1.5b.

Table 6.32.2a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | ATS_AC_STATUS_1, ATS_AC_STATUS_2, ATS_AC_STATUS_3, ATS_AC_STATUS_4, ATS_AC_STATUS_4 |
| DGN | 1FFADh, 1FFACh, 1FFABh, 1FF85h, xxxxxh |
| Default priority | 3 |
| Maximum broadcast gap | 5000 ms |
| Normal broadcast gap | 500 ms when active |
| Minimum broadcast gap | N/A |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.32.2b — Instance Field

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 to 2 | Transfer Switch instance | uint3 | - | 1 to 6 – Valid Instances 0 & 7 – Invalid Instances |
|  | 3 | Input/Output type | uint1 | - | 0b - Input 1b - Output |
|  | 4 to 6 | Source | uint3 | - | 000b - Primary (for inputs, typically generator) 001b - Secondary 111b - Invalid (no data) |
|  | 7 | Leg | uint1 | - | 0b - Leg1 1b - Leg2 |

6.32.3 AC Fault Configuration Status and Command

Fault Control configuration and status DGNs are defined for the transfer switch. These DGNs follow the formats as indicated in the following table.

Table 6.19.7 - DG Reference

| Name | DGN | Format | Table |
| --- | --- | --- | --- |
| ATS_ACFAULT_CONFIGURATION_STATUS_1 | 1FF84h | AC_CONFIGURATION_STATUS_1 | 6.1.6 |
| ATS_ACFAULT_CONFIGURATION_STATUS_2 | 1FF83h | AC_CONFIGURATION_STATUS_2 | 6.1.7 |
| ATS_ACFAULT_CONFIGURATION_COMMAND_1 | 1FF82h | ACFAULT_CONFIGURATION_COMMAND_1 | 6.1.10.3 |
| ATS_ACFAULT_CONFIGURATION_COMMAND_2 | 1FF81h | ACFAULT_CONFIGURATION_COMMAND_2 | 6.1.10.3 |

The status DGNs are broadcast on request. The command DGNs should be acknowledged with a NAK if necessary, and the corresponding status DGN.

If configuring one AC instance affects the configuration of other instances within the ATS, the node should respond with status information for all affected instances.

6.32.4 ATS Status

This DGN shows which source is currently active. Table 6.32.4a defines the DG attributes and Table 6.32.4b defines the signal and parameter attributes. The Instance corresponds to the ATS Instance of the ATS_AC_STATUS DGN Instances.

Table 6.32.4a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | ATS_STATUS |
| DGN | 1FFAAh |
| Default priority | 6 |
| Maximum broadcast gap | 5000 ms |
| Normal broadcast gap | on change |
| Minimum broadcast gap | 50 ms, see 3.2.4.2 |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.32.4b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 1 to 6 - Valid instances 0 & 7 to 255 – Invalid Instances |
| 1 | - | Source in use | uint8 | - | 0 - Primary 0 to 6 - Valid sources 253 - No source active |
| 2 | 0 to 1 | Mode | uint2 | - | 00b - Automatic 01b - Manual |

6.32.5 ATS Command

This DGN forces the transfer switch to a certain source, or releases it to automatically select.

The Instance corresponds to the ATS Instance of the ATS_AC_STATUS DGN Instances. The Source in Use corresponds to the Source bits of that Instance field as well. If the Mode is set to Automatic, the Source to Use is ignored.

Table 6.32.5a defines the DG attributes and Table 6.32.5b defines the signal and parameter attributes. The Instance corresponds to the ATS Instance of the ATS_AC_STATUS DGN Instances.

Table 6.32.5a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | ATS_COMMAND |
| DGN | 1FFA9h |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | ATS_STATUS, NAK |

Table 6.32.5b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 1 to 6 - Valid instances 0 & 7 to 255 – Invalid Instances |
| 1 | - | Source to use | uint8 | - | 0 - Primary 0 to 6 - Valid sources 253 - No source active |
| 2 | 0 to 1 | Mode | uint2 | - | 00b - Automatic 01b - Manual |

6.32.6 Service Points

The Intermediate Byte of the SPN indicates the Instance of the component with a fault. For the ATS this is the Transfer Switch Instance only. Note that the AC power faults are not reported with the DM-RV. The DM-RV is reserved for failures of the ATS components. Table 6.32.8 lists the allowable Service Points.

Table 6.32.8 — Service Points

| MSB | ISB | LSb | Description |
| --- | --- | --- | --- |
| 0 | 0-FFh | 0-7 | Node-Global Service Points. Combine the ISB and LSb to calculate. Values from 0-FFh are Standard SPNs – see Table 7.3 |
| 1 | Instance | 0 to 6 | Contactor. LSB indicates the input affected. |
| 2 | Instance | 0 to 6 | Contactor Neutral. LSB indicates the input affected. |

6.32.7 Alarms

Table 6.32.7 — Alarms

| Instance | Description |
| --- | --- |
| 1 | AC Source Switch Event |
| 2 | Impending AC Source Switch |
| 3 | AC Source Detected |
| 4 | AC Source Fault |

6.32.8 Test Profile

6.32.8.1 Profile 79-3A: Automatic Transfer Switch (Base) Profile

Reporting

| ID | Datum | Test | Required Response | Required Behavior |
| --- | --- | --- | --- | --- |
| 79-S-01 | ATS_STATUS | Instance Source in Use Mode | The ATS shall broadcast this DGN at least once every 5000ms. | N/A |
| 79-S-02 | ATS_STATUS | (Automatic Mode) Supply an AC source to a higher priority input. | ATS_STATUS Source in Use = higher priority input used. GENERIC_ALARM_STATUS instance = 1 | Switches to new higher priority AC Source |
| 79-S-03 | ATS_STATUS | (Automatic Mode) Supply an AC source to a lower priority input. | No Response. | Does not switch to new lower priority AC source. |
| 79-S-04 | ATS_STATUS | (Manual Mode) Supply an Ac source to higher priority input. | No Response. | Does not switch to new higher priority AC source. |
| 79-S-05 | ATS_STATUS_1 | Requirements Defined in AC Point Base Profile 02A-S-01 | Requirements Defined in AC Point Base Profile 02A-S-01 | Requirements Defined in AC Point Base Profile 02A-S-01 |

Command Response

| ID | Datum | Test | Required Response | Required Behavior |
| --- | --- | --- | --- | --- |
| 79-C-01 | ATS_COMMAND | Set to Manual Mode: Source to Use = Desired Source Mode = 01b | ATS_STATUS Source in Use = Desired Source Mode = 01b GENERIC_ALARM_STATUS instance =1 | Switches input source to the desired automatic switching. |
| 79-C-02 | ATS_COMMAND | Set to Auto Mode: Source to Use = N.A Mode = 00b | ATS_STATUS Source in Use = Source in Use Mode = 00b GENERIC_ALARM_STATUS, Instance = 1 | Switches to highest priority active input and enables automatic switching. |
| 79-C-03 | ATS_COMMAND | Set to Manual Mode: Source to Use = Invalid Source Mode = 01b | Reports with an acknowledgment DG with Acknowledgment Code = NAK and the DGN Acknowledged = ATS_COMMAND | N/A |