# Justification:

NEW REVISION “B” - 01/02/2023

New DGN’s request and protocol for wireless devices communications for the RV-C.

All DNG’s listed below are Tentative until this RV-C submission is approved.

REVISION B NOTES:

- Additional Changes and updates were made based on objections. See proposed changes in Yellow.

- Due to the number of changes the old OB submission will be move to the “Dead Submission Section.” On the RV-C. It can be reference here: http://www.rv-c.com/node/666

- Section 6.21.1 AC Output - Updated to Reflect Wireless Proposal (see end of the document)

- Section 6.42 DC System Disconnect & Bridge - Updated to Reflect Wireless Proposal (see end of the document)

- Table 7.2 Default Source Address Table Update - Updated to Reflect Wireless Proposal (see end of the document)

# External Interface

Table 6.44a — External Interface

| Device attribute | Value |
| --- | --- |
| Category | Appliances and Bridges |
| Default Source Address | 143 |
| Dynamic Address Range | 208-223 |
| Instance | Multi-Instance Each broker must have a unique Instance on the RV-C network. For each connection made by a broker, the broker assigns a connection instance which is unique only within that broker. Other brokers may use the same connection instance for their connections. |

For an external interface, a Broker is a device which connects to the RV-C network and includes an interface to communicate to devices that are not on the RV-C bus.  A Client is a device that connects to a broker via some non-RV-C protocol (e.g. Bluetooth, Wifi).  An Initiator is the device that starts the connection process; a Responder is a device which responds to the initiator's attempts to connect.

A broker may act in either role - for example, initiating connections to remote sensor devices, while responding to connection requests from cell phones.

Pairing is the attempt to establish a recurring connection; the term may be applied to protocols other than Bluetooth.

Table 6.44b – Connection Types

| Value | Definition |
| --- | --- |
| 0 | Invalid |
| 1 | Bluetooth LE |
| 2 | WiFi |
| 3 | Thread |
| 4 | Bluetooth |
| 5 | Ethernet |
| 6 | MODNet (DBTech, USPTO 90293345) |
| 60 | Unspecified Wireless Connection |
| 61 | Unspecified Wired Connection |

## Broker Status

Table 6.44.1a — DG Definition

| DG Attribute | Value |
| --- | --- |
| Name | BROKER_STATUS |
| DGN | 1FDB6h |
| Default Priority | 6 |
| Max Broadcast Gap | n/a |
| Normal Broadcast Gap | On Change |
| Minimum Broadcast Gap | 100 ms |
| Number of Frames | 1 |
| Ack requirements | None |

Table 6.44.1b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value Description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Broker Instance | Uint8 | - | 1-250 |
| 1 | - | Number Paired Devices | Uint8 | - | 0=none |
| 2 | - | Number Active Connections | Uint8 | - | 0=none |

## Broker Initiator Command

Table 6.44.2a — DG Definition

| DG Attribute | Value |
| --- | --- |
| Name | BROKER_INITIATOR_COMMAND |
| DGN | 1FDB5h |
| Default Priority | 6 |
| Max Broadcast Gap | n/a |
| Normal Broadcast Gap | As Needed |
| Minimum Broadcast Gap | 100 ms |
| Number of Frames | 1 |
| Ack requirements | BROKER _INITIATOR_STATUS |

Table 6.44.2b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value Description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Broker Instance | Uint8 | - | 1-250 |
| 1 | - | Connection Instance | Uint8 | - | 1-250 |
| 2 | 0-5 | Connection Type | Uint8 | - | See Table 6.44b |
| 2 | 6-7 | WRV-C Compatibility | Bit2 | - | 0 = Not WRV-C Compatible 1 = WRV-C Compatible. A valid WRV-C Connection Type must be specified. |
| 3 | 0-1 | Secured Connections | Bit2 | - | 00b = Request unsecured connections 01b = Request secured connections |
| 3 | 2-3 | Search for Device | Bit 2 | - | 00b = Stop Searching 01b = Begin Searching |
| 4 | - | Pairing | Uint8 | - | 0 = Cancel pairing request 1 = Begin pairing request 2 = Reset paired connections |

## Broker Initiator Status

Table 6.44.3a — DG Definition

| DG Attribute | Value |
| --- | --- |
| Name | BROKER _INITIATOR_STATUS |
| DGN | 1FDB4h |
| Default Priority | 6 |
| Max Broadcast Gap | n/a |
| Normal Broadcast Gap | On Change |
| Minimum Broadcast Gap | 100 ms |
| Number of Frames | 1 |
| Ack requirements | None |

Table 6.44.3b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value Description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Broker Instance | Uint8 | - | 1-250 |
| 1 | - | Connection Instance | Uint8 | - | 1-250 |
| 2 | 0-5 | Connection Type | Uint8 | - | See Table 6.44b |
| 2 | 6-7 | WRV-C Compatibility | Bit2 | - | 0 = Not WRV-C Compatible 1 = WRV-C Compatible. A valid WRV-C Connection Type must be specified. |
| 3 | 0-1 | Secured Connections | Bit2 | - | 00b = Unsecured connections supported 01b = Secured connections supported See below. |
| 3 | 2-3 | Searching for Devices | Bit 2 | - | 00b = Not Searching 01b = Searching in Progress |
| 4 | - | Pairing | Uint8 | - | 0 = Device Not Found 1 = Device Found, Not Pairing 2 = Device Found, Pairing 3 = Device Pairing in Progress 4 = Device Paired |

For insecure connections, it is the responsibility of the Broker to serve as a firewall and prevent the connection from being used to access the Broker or the RV-C network in an insecure manner. Insecure connections shall be limited to communicating a well-defined set of data items, the spoofing or interception of which have minimal security consequences. It should not be possible to use an insecure connection to download firmware, transmit or receive arbitrary RV-C data, or otherwise bypass the physical security of the network.

## Broker Responder Command

Table 6.44.4a — DG Definition

| DG Attribute | Value |
| --- | --- |
| Name | BROKER_RESPONDER_COMMAND |
| DGN | 1FDB3h |
| Default Priority | 6 |
| Max Broadcast Gap | n/a |
| Normal Broadcast Gap | As Needed |
| Minimum Broadcast Gap | 100 ms |
| Number of Frames | 1 |
| Ack requirements | BROKER _RESPONDER_STATUS |

Table 6.44.4b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value Description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | Uint8 | - | 1-250 |
| 1 | - | Connection Instance | Uint8 | - | 1-250 |
| 2 | 0-5 | Connection Type | Uint8 | - | See Table 6.44b |
| 2 | 6-7 | WRV-C Compatibility | Bit2 | - | 0 = Not WRV-C Compatible 1 = WRV-C Compatible. A valid WRV-C Connection Type must be specified. |
| 3 | 0-1 | Visibility | Bit2 | - | 00b = hidden 01b = visible |
| 3 | 2-3 | Secured Connections | Bit2 | - | 00b = Accept unsecured connections 01b = Deny unsecured connections |
| 4 | - | Pairing | Uint8 | - | 0 = Reject pairing request 1 = Confirm pairing request 2 = Reset paired connections |

## Broker Responder Status

Table 6.44.5a — DG Definition

| DG Attribute | Value |
| --- | --- |
| Name | BROKER _RESPONDER_STATUS |
| DGN | 1FDB2h |
| Default Priority | 6 |
| Max Broadcast Gap | n/a |
| Normal Broadcast Gap | On Change |
| Minimum Broadcast Gap | 100 ms |
| Number of Frames | 1 |
| Ack requirements | None |

Table 6.44.5b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value Description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | Uint8 | - | 1-250 |
| 1 | - | Connection Instance | Uint8 | - | 1-250 |
| 2 | 0-5 | Connection Type | Uint8 | - | See Table 6.44b |
| 2 | 6-7 | WRV-C Compatibility | Bit2 | - | 0 = Not WRV-C Compatible 1 = WRV-C Compatible. A valid WRV-C Connection Type must be specified. |
| 3 | 0-1 | Visibility | Bit2 | - | 00b = hidden 01b = visible |
| 3 | 2-3 | Secured Connections | Bit2 | - | 00b = Unsecured connections supported 01b = Secured connections supported |
| 4 | - | Pairing | Uint8 | - | 0 = No Pairing Requests 1 = Pairing request received 2 = Pairing in Progress 3 = Connection is Paired |
| 5 | - | Last Command | Uint8 | - | 0 = Reset Paired Connections 1 = Confirm Pair Request 2 = Reject Pair Request 3 = Change to Hidden 4 = Change to Visible |

These DGNs allow the communication of text information associated with a connection, such as the Bluetooth name.  Transmitting the text requires one BROKER_CONNECTION_STATUS/COMMAND message to indicate the length of the text, and (length / 5) BROKER_CONNECTION_TEXT_STATUS/COMMAND messages to transfer the text contents.  The maximum text length is 255 characters.

Note that these DGNs may apply differently in various contexts.  For example, they may be used by a broker to inform a UI device of possible connections (e.g. the results of a Bluetooth scan), or by a UI device to inform a broker to connect to a specific client (e.g. a URL for an Ethernet connection).

## Broker Connection Command

Table 6.44.6a — DG Definition

| DG Attribute | Value |
| --- | --- |
| Name | BROKER_CONNECTION_COMMAND |
| DGN | 1FDB1h |
| Default Priority | 6 |
| Max Broadcast Gap | n/a |
| Normal Broadcast Gap | As Needed |
| Minimum Broadcast Gap | 100 ms |
| Number of Frames | 1 |
| Ack requirements | BROKER_CONNECTION_STATUS |

Table 6.44.6b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value Description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | Uint8 | - | 1-250 |
| 1 | - | Connection Instance | Uint8 | - | 1-250 |
| 2 | - | Connection Text Identifier Length | Uint8 | - | 0= None,… Max 253 |

## Broker Connection Status

Table 6.44.7a — DG Definition

| DG Attribute | Value |
| --- | --- |
| Name | BROKER_CONNECTION_STATUS |
| DGN | 1FDB0h |
| Default Priority | 6 |
| Max Broadcast Gap | n/a |
| Normal Broadcast Gap | On Change |
| Minimum Broadcast Gap | 100 ms |
| Number of Frames | 1 |
| Ack requirements | none |

Table 6.44.7b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value Description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | Uint8 | - | 1-250 |
| 1 | - | Connection Instance | Uint8 | - | 1-250 |
| 2 | - | Substring Index | Uint8 | - | 0-250 |
| 3 | - | Connection Text Identifier Length | Uint8 | - | 0= None,… Max 253 |

If this DGN is requested, only this DGN shall be transmitted.  The BROKER_CONNECTION_TEXT_STATUS must be requested separately, but such request will cause both DGNs to be transmitted.

## Broker Connection Text Command

Table 6.44.8a — DG Definition

| DG Attribute | Value |
| --- | --- |
| Name | BROKER_CONNECTION_TEXT_COMMAND |
| DGN | 1FDAFh |
| Default Priority | 6 |
| Max Broadcast Gap | n/a |
| Normal Broadcast Gap | As Needed |
| Minimum Broadcast Gap | 100 ms |
| Number of Frames | 1 |
| Ack requirements | BROKER_CONNECTION_TEXT_STATUS |

Table 6.44.8b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value Description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | Uint8 | - | 1-250 |
| 1 | - | Connection Instance | Uint8 | - | 1-250 |
| 2 | - | Substring Index | Uint8 | - | 0-250 |
| 3 | - | ASCII Char 1 | Uint8 | - | 0 = beyond end of text |
| 4 | - | ASCII Char 2 | Uint8 | - | 0 = beyond end of text |
| 5 | - | ASCII Char 3 | Uint8 | - | 0 = beyond end of text |
| 6 | - | ASCII Char 4 | Uint8 | - | 0 = beyond end of text |
| 7 | - | ASCII Char 5 | Uint8 | - | 0 = beyond end of text |

This DGN shall be preceded by the BROKER_CONNECTION_COMMAND/STATUS.  
Exactly as many BROKER_CONNECTION_TEXT_COMMAND/STATUS packets are transmitted as necessary to encode the full string.

## Broker Connection Text Status

Table 6.44.9a — DG Definition

| DG Attribute | Value |
| --- | --- |
| Name | BROKER_CONNECTION_TEXT_STATUS |
| DGN | 1FDAEh |
| Default Priority | 6 |
| Max Broadcast Gap | n/a |
| Normal Broadcast Gap | On Change |
| Minimum Broadcast Gap | 100 ms |
| Number of Frames | 1 |
| Ack requirements | none |

Table 6.44.9b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value Description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | Uint8 | - | 1-250 |
| 1 | - | Connection Instance | Uint8 | - | 1-250 |
| 2 | - | Substring Index | Uint8 | - | 0-248 |
| 3 | - | ASCII Char 1 | Uint8 | - | 0 = beyond end of text |
| 4 | - | ASCII Char 2 | Uint8 | - | 0 = beyond end of text |
| 5 | - | ASCII Char 3 | Uint8 | - | 0 = beyond end of text |
| 6 | - | ASCII Char 4 | Uint8 | - | 0 = beyond end of text |
| 7 | - | ASCII Char 5 | Uint8 | - | 0 = beyond end of text |

This DGN shall be preceded by the BROKER_CONNECTION_COMMAND/STATUS.  
Exactly as many BROKER_CONNECTION_TEXT_COMMAND/STATUS packets are transmitted as necessary to encode the full string.

# 6.21.1 AC Output Original Section:

6.21.1 AC Output Introduction

When measuring AC power from a Generator, Inverter, or Transfer Switch, specific DGNs are defined. AC from other sources, such as the Shore Cord, is reported as a Generic AC Source. The following formats apply (see Table 6.21.1a).

Table 6.21.1a — Generic AC Source definition

| Device attribute | Value |
| --- | --- |
| Category | Power components |
| Default Source Address | 139 |
| Dynamic Address Range | 128 to 143 |
| Instance | Multiple |

AC output is reported using the standard AC Point formats (see Section 6.1). The Instance field is defined in Table 6.21.1b. Note that each of these DGNs may have to be transmitted multiple times to provide information on each line.

# Proposed Changes in Yellow:

…

Table 6.21.1a —  Generic AC Source definition

| Device attribute | Value |
| --- | --- |
| Category | Power components |
| Default Source Address | 140 |
| Dynamic Address Range | 128-143 |
| Instance | Multi-Instance |

…

# 6.42 DC System Disconnect and Bridge

# Original Section:

6.42 DC System Disconnect and Bridge

6.42.1 Introduction

These DGNs are for a DC System Disconnect. This is typically a solenoid or similar cut-off switch that control the DC power to a substantial portion of the RV. It may also bridge two or more DC systems, such as the chassis and house batteries. The precise function of the device is not explicit in the DGN - there is generally no way to identify the purpose of a circuit in RV-C. The following formats apply (see Table 6.42).

Table 6.42 — DC Disconnect/Bridge

| Device attribute | Value |
| --- | --- |
| Category | Power components |
| Default Source Address | 143 |
| Dynamic Address Range | 128 to 143 |
| Instance | Multiple |

Each circuit is identified with an Instance from 1 to 250. In practice multiple instances are likely to be contained in a single controller.

# Proposed Changes in Yellow:

…

Table 6.42 — DC Disconnect/Bridge

| Device attribute | Value |
| --- | --- |
| Category | Power components |
| Default Source Address | 139 |
| Dynamic Address Range | 128-143 |
| Instance | Multi-Instance |

…

# 7.2 Default Source Addresses

# Original Section:

Table 7.2 Default Source Addresses

| Product | DSA | Preferred Dynamic Address Range | Dynamic Address Group | Section |
| --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... |
| DC Motor Controller | 138 | 128-143 | Power Components | 6.27 |
| DC Disconnect | 139 | 144-159 | Appliances and Bridges | 6.44 |
| Generic AC Source | 140 | 128-143 | Power Components | 6.21 |
| Solar Charge Controller | 141 | 128-143 | Power Components | 6.45 |
| Roof Fan | 142 | 192-207 | Comfort Systems | 6.46 |
| External Interface | 143 | 128-143 | Power Components | 6.42 |
| Generic Alarm | 144 | 160-175 | Sensors | 6.47 |
| Window Control | 145 | 176-191 | Mechanical Components | 6.40 |
| DC Load | 146 | 128-143 | Power Components | 6.23 |
| ... | ... | ... | ... | ... |

# Proposed Changes in Yellow:

Table 7.2 Default Source Addresses

| Product | DSA | Preferred Dynamic Address Range | Dynamic Address Group | Section |
| --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... |
| DC Motor Controller | 138 | 128-143 | Power Components | 6.27 |
| DC Disconnect | 139 | 128-143 | Power Components | 6.42 |
| Generic AC Source | 140 | 128-143 | Power Components | 6.21 |
| Solar Charge Controller | 141 | 128-143 | Power Components | 6.45 |
| Roof Fan | 142 | 192-207 | Comfort Systems | 6.46 |
| External Interface | 143 | 208-223 | Appliances and Bridges | 6.44 |
| Generic Alarm | 144 | 160-175 | Sensors | 6.47 |
| ... | ... | ... | ... | ... |