6.45.2 Solar Controller Status

The controller status DGN describes the general operating status of the Solar Controller on a particular DC Source (Battery Bank). Table 6.45.2a defines the DG attributes and Table 6.45.2b defines the signal and parameter attributes.

Table 6.45.2a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | SOLAR_CONTROLLER_STATUS |
| DGN | 1FEB3h |
| Default priority | 6 |
| Maximum broadcast gap | 5000 ms |
| Normal broadcast gap | 5000 ms or on change |
| Minimum broadcast gap | 500 ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.45.2b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - |  |
| 1 to 2 | - | Charge voltage | uint16 | Vdc | see Table 5.3 Control voltage: The voltage desired to be delivered to the battery. |
| 3 to 4 | - | Charge current | uint16 | Adc | see Table 5.3 Control current: The current desired to be delivered to the battery. |
| 5 | - | Charge current percent of maximum | uint8 | % | see Table 5.3 Control current as a percent of the maximum. |
| 6 | - | Operating state | uint8 | - | Specifies the current operating state of the controller. see table 6.5.5b |
| 7 | 0 to 1 | Power-up state | uint2 | - | 00b - controller disabled 01b - controller enabled |
|  | 2 to 3 | Clear history | uint2 | - | 00b - History not clear 01b - History cleared |
|  | 4 to 7 | Force Charge | uint4 | - | 0 - Charging is NOT forced 1 - Force charge to bulk 2 - Force charge to float |