# Justification:

Provide a method for solar controllers to report their supported battery chemistries.

# Proposed Changes (in Red):

6.45 Solar Charge Controller

6.45.1 Introduction

Solar Charge Controller(s) are associated with Solar Panels to provide features such as multi-stage battery charging. There may be more than It one charge controller associated with a single DC source (battery) to increase capacity. The following formats apply (see Table 6.45.1).

Table 6.45.1 — Solar definition

| Device attribute | Value |
| --- | --- |
| Category | Power components |
| Default Source Address | 141 |
| Dynamic Address Range | 128 to 143 |
| Instance | Multiple |

6.45.2 Solar Controller Status

The controller status DGN describes the general operating status of the Solar Controller on a particular DC Source (Battery Bank). Table 6.45.2a defines the DG attributes and Table b defines the signal and parameter attributes.

Table 6.45.2a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | SOLAR_CONTROLLER_STATUS |

| DGN | 1FEB3h |
| --- | --- |
| Default priority | 6 |
| Maximum broadcast gap | 5000 ms |
| Normal broadcast gap | 5000 ms or on change |
| Minimum broadcast gap | 500 ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.45.2b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - |  |
| 1 to 2 | - | Charge voltage | uint16 | Vdc | see Table 5.3 Control voltage: The voltage desired to be delivered to the battery. |
| 3 to 4 | - | Charge current | uint16 | Adc | see Table 5.3 Control current: The current desired to be delivered to the battery. |
| 5 | - | Charge current percent of maximum | uint8 | % | see Table 5.3 Control current as a percent of the maximum. |
| 6 | - | Operating state | uint8 | - | Specifies the current operating state of the controller. see table 6.5.5b |
| 7 | 0 to 1 | Power-up state | uint2 | - | 00b - controller disabled 01b - controller enabled |
|  | 2 to 3 | Clear history | uint2 | - | 00b - History not clear 01b - History cleared |
|  | 4 to 7 | Force Charge | uint4 | - | 0 - Charging is NOT forced 1 - Force charge to bulk 2 - Force charge to float |

6.45.3 Solar Controller Status 2

The controller status DGN describes the general operating status of the Solar Controller on a particular DC Source (Battery Bank). Table 6.45.3a defines the DG attributes and Table 6.45.3b defines the signal and parameter attributes.

Table 6.45.3a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | SOLAR_CONTROLLER_STATUS_2 |
| DGN | 1FE85h |
| Default priority | 6 |
| Maximum broadcast gap | 5000 ms |
| Normal broadcast gap | 5000 ms or on change |
| Minimum broadcast gap | 500 ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.45.3b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - |  |
| 1 to 2 | - | Rated battery voltage | uint16 | Vdc | Maximum battery voltage the solar charge controller can handle on its battery terminals |
| 3 to 4 | - | Rated charging current | uint16 | Adc | Maximum current that the solar charge controller is capable of outputting to the battery. |
| 5 | - | Supported Battery Types | Bitmap | - | Bitmap of battery types the solar controller can be set to using SOLAR_CONTROLLER _CONFIGURATION _COMMAND, “Battery type” field. Follows the order and type values of Table 6.5.5b, byte 7. A zero value for a bit indicates the corresponding battery type is not supported; a one value for a bit indicates the corresponding battery type is supported. Bit 0 – Type 0 – Flooded battery type Bit 1 – Type 1 – Gel battery type Bit 2 – Type 2 – AGM battery type Bit 3 – Type 3 – LiFePh battery type Bits 4-6 – Battery types 4, 5 and 6 (not yet defined in Table 6.5.5b; reserved for future battery types; set as 0) Bit 7 – Always 0; Ensures that 0xFF or 0xFE value is correctly parsed. |
| 6 | - | Supported Battery Types (Continued) | Bitmap | - | Same description as byte 5 above. Bits 0-4 – Battery types 7, 8, 9, 10 and 11 (not yet defined in Table 6.5.5b; reserved for future battery types; set as 0) Bit 5 – Type 12 – “Vendor-defined proprietary type” 1 Bit 6 – Type 13 – “Vendor-defined proprietary type” 2 Bit 7 – Always 0; Ensures that 0xFF or 0xFE value is correctly parsed. |
| 7 | 0 to 1 | Vendor-Defined Proprietary Type 1 – User Defined Charge Parameters | uint2 | - | 0 – This battery type does not support user definable charge parameters. 1 - This battery type supports user definable charge parameters. If this field is 0 and the solar controller is set to battery type 12, “Vendor-Defined Proprietary Type 1”, the solar controller will not accept commands to alter battery charge parameters, such as those in CHARGER_CONFIGURATION_COMMAND_3 or CHARGER_CONFIGURATION_COMMAND_4. If this field is 1, the solar controller must accept commands to configure battery charging parameters. |
|  | 2 to 3 | Vendor-Defined Proprietary Type 2 – User Defined Charge Parameters | uint2 | - | Same as above, but for battery type 13, “Vendor-Defined Proprietary Type 2”. |

6.45.4 Solar Controller Status 3

The controller status DGN describes the general operating status of the Solar Controller on a particular DC Source (Battery Bank). Table 6.45.4a defines the DG attributes and Table 6.45.4b defines the signal and parameter attributes.

Table 6.45.4a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | SOLAR_CONTROLLER_STATUS_3 |
| DGN | 1FE84h |
| Default priority | 6 |
| Maximum broadcast gap | 5000 ms |
| Normal broadcast gap | 5000 ms or on change |
| Minimum broadcast gap | 500 ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.45.4b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - |  |
| 1 to 2 | - | Rated solar input voltage | uint16 | Vdc | Maximum voltage the solar charge controller can accept on the solar input terminals. |
| 3 to 4 | - | Rated solar input current | uint16 | Vdc | Maximum current the solar charge controller can accept into the solar input. |
| 5 to 6 |  | Rated solar over-power | uint16 | W | How much the solar array can be oversized without causing damage to the solar charge controller |