# Proposal for BATTERY DGN Additions

The following additions are proposed to the BATTERY DGNs to help load management and prevent unnecessary DC disconnects when possible. DGNs are designed to mirror existing DC_SOURCE_STATUS_4 which is used in similar manner to control charging devices.

Also proposing consideration to rename BATTERY_STATUS_4 to BATTERY_CHARGE_CONTROL which better describes their purposes.

6.49.20 Battery Load Control

Table 6.49.20a defines the DG attributes, and Table 6.49.Xb defines the signal and parameter attributes. This DGN is a mirror of the DC_SOURCE_LOAD_CONTROL to allow individual battery management in a multi-battery DC bank systems.

Table 6.49.20a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | BATTERY_LOAD_CONTROL |
| DGN | 1FDA8 |
| Default priority | 6 |
| Maximum broadcast gap | 5000 ms |
| Normal broadcast gap | On Change |
| Minimum broadcast gap | 100 ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.49.20b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Battery Instance | uint8 | - | 1-250 |
| 1 | - | DC Instance | uint8 | - | DC Instance this Battery is associated with. |
| 2 | - | Desired Load State | uint8 | - | Specifies desired state for load devices, such as inverters. 0 – Normal Operation (default state) 1 – Stop or reduce all loads to the minimum. 2 – Stop non-critical loads, only keep low or critical loads. |
| 3 to 4 | - | Desired Min Voltage | uint16 | - | see Table 5.3 The desired minimal voltage the battery is targeting during discharge to prevent Low Voltage Disconnect. |
| 5 to 6 | - | Desired Max Current | uint16 | - | see Table 5.3 The desired maximum current the battery is targeting during discharge to prevent Overload Disconnect. |