6.49 Battery Update

JUSTIFICATION:

Some time values were consistent with the definition of a given message, as well as with conventions used in RV-C document.  Example, calling for max Tx rate for a message that is defined as Pull-only model.

The Battery sub team reviewed this document, including a question of if the largely Pull-only model should be retained. It was agreed to keep the pull-only model as in most cases Battery details are used in advanced service screens, as opposed to active battery bank aggregation.   The battery sub team does however support the concept of modifiable Tx rates for messages, and the including of a new RV-C capability to allow for some Battery messages to be modified from a Pull model to a Push model at runtime.  It was felt such an approach would retain the present approach – minimizing CAN bus load, while allowing for Battery Aggregation using the RV-C Battery messages if a given node choses to implement that.

WAS:

6.49.4 Battery Status 4

Table 6.49.4a defines the DG attributes, and Table 6.49.4b defines the signal and parameter attributes.

See DC_SOURCE_STATUS_4 for additional notes regarding this message.

Table 6.49.4a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | BATTERY_STATUS_4 |
| DGN | 1FE92h |
| Default priority | 6 |
| Maximum broadcast gap | 100ms |
| Normal broadcast gap | On Change, On Request |
| Minimum broadcast gap | 50 ms, see 3.2.4.2 |
| Number of frames | 1 |
| ACK requirements | None |

CHANGE:

6.49.4 Battery Status 4

Table 6.49.4a defines the DG attributes, and Table 6.49.4b defines the signal and parameter attributes.

See DC_SOURCE_STATUS_4 for additional notes regarding this message.

Table 6.49.4a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | BATTERY_STATUS_4 |
| DGN | 1FE92h |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | On Change, On Request |
| Minimum broadcast gap | 50 ms, see 3.2.4.2 |
| Number of frames | 1 |
| ACK requirements | None |

WAS:

6.49.6 Battery Status 6

See DC_SOURCE_STATUS_6 for additional notes. Table 6.49.6a defines the DG attributes, and Table 6.49.6b defines the signal and parameter attributes.

.

Table 6.49.6a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | BATTERY_STATUS_6 |
| DGN | 1FE90h |
| Default priority | 6 |
| Maximum broadcast gap | 100ms |
| Normal broadcast gap | On Change, On Request |
| Minimum broadcast gap | 50 ms, see 3.2.4.2 |
| Number of frames | 1 |
| ACK requirements | None |

CHANGE:

6.49.6 Battery Status 6

See DC_SOURCE_STATUS_6 for additional notes. Table 6.49.6a defines the DG attributes, and Table 6.49.6b defines the signal and parameter attributes.

.

Table 6.49.6a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | BATTERY_STATUS_6 |
| DGN | 1FE90h |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | On Change, On Request |
| Minimum broadcast gap | 50 ms, see 3.2.4.2 |
| Number of frames | 1 |
| ACK requirements | None |

WAS:

6.49.7 Battery Status 7

Table 6.49.7a defines the DG attributes, and Table 6.49.7b defines the signal and parameter attributes.

Table 6.49.7a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | BATTERY_STATUS_7 |
| DGN | 1FE8Fh |
| Default priority | 6 |
| Maximum broadcast gap | 100ms |
| Normal broadcast gap | On Request |
| Minimum broadcast gap | 50 ms, see 3.2.4.2 |
| Number of frames | 1 |
| ACK requirements | None |

CHANGE:

6.49.7 Battery Status 7

Table 6.49.7a defines the DG attributes, and Table 6.49.7b defines the signal and parameter attributes.

Table 6.49.7a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | BATTERY_STATUS_7 |
| DGN | 1FE8Fh |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | On Request |
| Minimum broadcast gap | 50 ms, see 3.2.4.2 |
| Number of frames | 1 |
| ACK requirements | None |

FORMAT CLEANUP:

6.49.12 Battery Status 12

Table 6.49.12a defines the DG attributes, and Table 6.49.12b defines the signal and parameter attributes.

Table 6.49.12a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | BATTERY_STATUS_12 |
| DGN | 1FDF3h |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | On Request |
| Minimum broadcast gap | 50 ms, see 3.2.4.2 |
| Number of frames | 1 |
| ACK requirements | None |

WAS:

6.49.15 Battery Summary

The Battery Summary DG communicates details about a given battery and its relationship to an associated DC_SOURCE instance (if assigned). Other details include number of Modules within a battery and (optionally) series connections of individual Batteries to support the associated higher DC_SOURCE bus voltages.

Battery Summary may also be used to communicate the available batteries in a system. A global request for the BATTERY_SUMMARY DG shall cause all batteries to reply.

Table 6.49.15a defines the DG attributes, and Table 6.49.15b defines the signal and parameter attributes.

Table 6.49.15a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | BATTERY_SUMMARY |
| DGN | 1FDF1h |
| Default priority | 6 |
| Maximum broadcast gap | 100ms |
| Normal broadcast gap | On Change, On request |
| Minimum broadcast gap | 50 ms, see 3.2.4.2 |
| Number of frames | 1 |
| ACK requirements | BATTERY_SUMMARY, NAK |

6.49.15 Battery Summary

The Battery Summary DG communicates details about a given battery and its relationship to an associated DC_SOURCE instance (if assigned). Other details include number of Modules within a battery and (optionally) series connections of individual Batteries to support the associated higher DC_SOURCE bus voltages.

Battery Summary may also be used to communicate the available batteries in a system. A global request for the BATTERY_SUMMARY DG shall cause all batteries to reply.

Table 6.49.15a defines the DG attributes, and Table 6.49.15b defines the signal and parameter attributes.

Table 6.49.15a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | BATTERY_SUMMARY |
| DGN | 1FDF1h |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | On Change, On request |
| Minimum broadcast gap | 50 ms, see 3.2.4.2 |
| Number of frames | 1 |
| ACK requirements | BATTERY_SUMMARY, NAK |

WAS:

6.49.16 Cell Detail

A battery may receive the BATTERY_COMMAND DGN asking for cell level details for one (or all) of its banks. When making a request, ‘Battery Instance’ and ‘Module Instance’ are filled in with the requested details. Upon reception of this message, the associated battery will respond with a series of Cell Detail messages, one for each cell until all cells within the requested module is communicated. After all cells within a module are communicated, transmission will stop.

If this message is received with an invalid module index, NAK is sent back. NAK is also sent back if the requested node does not support Cell Level messaging, or if the requested battery index is incorrect.

Table 6.49.16a defines the DG attributes, and Table 6.49.16b defines the signal and parameter attributes.

Table 6.49.16a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | CELL_DETAIL |
| DGN | 1FDDDh |
| Default priority | 7 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | On Request |
| Minimum broadcast gap | 25ms |
| Number of frames | 1 |
| ACK requirements | NAK, BATTERY_CELL_DETAIL |

CHANGE:

6.49.16 Cell Detail

A battery may receive the BATTERY_COMMAND DGN asking for cell level details for one (or all) of its banks. When making a request, ‘Battery Instance’ and ‘Module Instance’ are filled in with the requested details. Upon reception of this message, the associated battery will respond with a series of Cell Detail messages, one for each cell until all cells within the requested module is communicated. After all cells within a module are communicated, transmission will stop.

If this message is received with an invalid module index, NAK is sent back. NAK is also sent back if the requested node does not support Cell Level messaging, or if the requested battery index is incorrect.

Table 6.49.16a defines the DG attributes, and Table 6.49.16b defines the signal and parameter attributes.

Table 6.49.16a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | CELL_DETAIL |
| DGN | 1FDDDh |
| Default priority | 7 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | On Request |
| Minimum broadcast gap | 50 ms, see 3.2.4.2 |
| Number of frames | 1 |
| ACK requirements | NAK, BATTERY_CELL_DETAIL |