# Proposal for DC Source Configuration Command 2 Additions

The following additions are proposed to the DC Source Configuration Command 2 DGN

6.5.19 DC Source Configuration Command 2

Table 6.5.19a defines the DG attributes, and Table 6.5.19b defines the signal and parameter attributes.

Table 6.5.19a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | DC_SOURCE_CONFIGURATION_COMMAND_2 |
| DGN | 1FDF4h |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | ACK Always if Clear History, Set Capacity or Reset Battery Health included. NAK, DC_SOURCE_CONFIGURATION_STATUS _2, DC_SOURCE_STATUS_4 if battery type changed. |

Table 6.5.19b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | DC Instance | uint8 | - | see table 6.5.2b |
| 1 | 0-1 | Clear History | uint2 | - | 00b — No action 01b — Clear history |
|  | 2-3 | Set Capacity to 100% | uint2 | - | When the battery or battery bank is known to be fully charged, this command is used to set the state of charge to 100%. 00b — No action 01b — Set capacity to 100% |
|  | 4-5 | Reset Battery Health | uint2 | - | When battery is replaced, this command is used to reset battery state of health and any other parameters related to battery health. Generally, a clear history command should be issued at the same time. 00b — No action 01b — Reset battery health to 100% |
| 2 to 3 | - | Charged Voltage | uint16 | Vdc | See Table 5.3 The battery voltage must be above this voltage level to consider the battery to be fully charged |
| 4 | - | Shunt Voltage | uint8 | mV | Rated voltage of the shunt Precision = 1 mV Value range = 1 to 253 mV |
| 5 to 6 | - | Shunt Current | uint16 | Adc | See Table 5.3 Rated current of the shunt |
| 7 | 0-3 | Battery Type | uint4 | - | Sets the battery type. 0 – Flooded 1 – Gel 2 – AGM 3 – Lithium-Iron-Phosphate 12-13 – Reserved for Vendor-defined proprietary types. |