# Proposal for DC_SOURCE DGN Additions

The following additions are proposed to the DC_SOURCE DGNs to help load management and prevent unnecessary DC disconnects when possible. DGNs are designed to mirror existing DC_SOURCE_STATUS_4 which is used in similar manner to control charging devices.

Also proposing consideration to rename DC_SOURCE_STATUS_4 to DC_SOURCE_CHARGE_CONTROL which better describes their purposes.

6.5.24 DC Source Load Control

DC Source Load Control allows a smart battery or a BMS to signal its discharge power limits in real time, in an attempt to prevent or delay a DC disconnect event due to overcurrent or undervoltage conditions. It also allows for future development of intelligent load distribution where loads can be categorized as critical or non-critical, with room to create additional categories as needed.

A typical use scenario in an RV is where the RV-C compliant Inverter would stop 120VAC loads, which are often high power and non-critical when battery is getting low, but still has certain reserve which can be allocated to more critical DC loads such as lights, fans, DC compressor fridge, etc. This would prevent or delay imminent loss of DC power by delaying Low Battery Disconnect from the BMS.

Initially we only expect to use Desired Load State 0 and 1, only with RV-C enabled 120VAC inverters.
In the future more intelligent power distribution devices can use additional Desired Load States to select between critical and non-critical loads.

Table 6.5.24a defines the DG attributes, and Table 6.5.Xb defines the signal and parameter attributes.

Table 6.5.24a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | DC_SOURCE_LOAD_CONTROL |
| DGN | 1FDA9 |
| Default priority | 6 |
| Maximum broadcast gap | 5000 ms |
| Normal broadcast gap | On Change |
| Minimum broadcast gap | 100 ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.5.24b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | DC Instance | uint8 | - | see table 6.5.2b |
| 1 | - | Device Priority | uint8 | - | see table 6.5.2b |
| 2 | - | Desired Load State | uint8 | - | Specifies desired state for load devices, such as inverters. 0 – Normal Operation (default state) 1 – Stop or reduce all loads to the minimum. 2 – Stop non-critical loads, only keep low or critical loads. |
| 3 to 4 | - | Desired Min Voltage | uint16 | - | see Table 5.3 The desired minimal voltage the battery is targeting during discharge to prevent Low Voltage Disconnect. |
| 5 to 6 | - | Desired Max Current | uint16 | - | see Table 5.3 The desired maximum current the battery is targeting during discharge to prevent Overload Disconnect. |