Justification:  Purpose of submission is to expand the list of shutdown reasons under the DC Comp Driver status 1

Table 6.50.1b — Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Value Definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Device Instance | uint8 | - | 1-250, FFh for non-instanced devices. |
| 1 | - | Driver Index | uint8 | - | 1-250, FFh is acceptable for devices with a single driver. |
| 2 to 3 | - | Voltage | uint16 | Vdc | see Table 5.3 |
| 4 to 5 | - | Current | uint16 | Adc | see Table 5.3 |
| 6 | 0 to 1 | Output Status | uint2 | - | 00b - Off 01b - On |
|  | 2 to 3 | Desired Status | uint2 | - | 00b - Off 01b - On |
|  | 4 to 5 | Shutdown Status | uint2 | - | 00b - Ok 01b - Output Disabled (see Byte 7) |
|  | 6 to 7 | Reset Type | uint2 | - | 00b - Manual 01b - Automatic |
| 7 | - | Reason for Shutdown | uint8 | - | 0 = Output Not Disabled 1 = Over Current 2 = Short Circuit 3 = Over Voltage 4 = Under Voltage 5 = Under Current 6 = Over Temperature 7 = Hardware Disabled 8 = Driver Fault 9 = Max On time exceeded 10 = Fault Recovery Period Active |