Justification:  Dometic NRX refrigerators currently support eco and freezer modes.  We are working to connect these refrigerators to RV-C, and we want to give our customer’s multiplex systems / user APPs / Winnebago Connect, etc. the ability to see the status of all of the modes that are available on the refrigerator’s HMI (Human Machine Interface).

6.52.2 Refrigerator Status

Table 6.52.2a defines the DG attributes and Table 6.52.2b defines the signal and parameter attributes.

Table 6.52.2a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | REFRIGERATOR_STATUS |
| DGN | 1FDD3h |
| Default priority | 6 |
| Maximum broadcast gap | 5000 |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | 500 ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.52.2b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 to 5 | Instance | uint6 | - | 1 to 60 |
|  | 6 to 7 | Cavity | uint2 | - | 00b - fridge 01b - freezer |
| 1 | 0 to 1 | Light | uint2 | - | 00b – all off 01b – all on |
|  | 2 to 3 | Door Switch | uint2 | - | 00b – closed 01b - open |
| 2 to 3 | - | Current Temperature | uint16 | °C | See Table 5.3 |
| 4 to 5 | - | Set Temperature | uint16 | °C | See Table 5.3 |
| 6 | 0 to 3 | Fuel Source | uint4 | - | 000b – Gas 001b – DC Voltage 010b – AC Voltage |
|  | 4 to 7 | Refrigerator Mode | uint4 | - | 0000b – off 0001b – on 0010b – night 0011b – eco 0100b – freezer |
| 7 | - | Compressor Speed | uint8 | % | See Table 5.3 |