Justification:  Dometic NRX refrigerators currently support night, eco and freezer modes.  We are working to connect these refrigerators to RV-C and we want to give our customer’s multiplex systems / user APPs / Winnebago Connect, etc. the ability to set all of the modes that are available on the refrigerator’s HMI (Human Machine Interface).

6.52.3 Refrigerator Command

Table 6.52.3a defines the DG attributes and Table 6.52.3b defines the signal and parameter attributes. This DGN triggers refrigerator actions.

Table 6.52.3a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | REFRIGERATOR_COMMAND |
| DGN | 1FDD2h |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | NAK, REFRIGERATOR_STATUS |

Table 6.52.3b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 to 5 | Instance | uint6 | - | See Table 5.3 |
|  | 6 to 7 | Cavity | uint2 | - | See Table 5.3 |
| 1 | 0 to 1 | Light | uint2 | - | 00b – turn all off 01b – turn all on |
| 4 to 5 | - | Set Temperature | uint16 | °C | See Table 5.3 |
| 6 | 0 to 3 | Fuel Source | uint4 | - | 0000b – Gas 0001b – DC Voltage 0010b – AC voltage |
|  | 4 to 7 | Refrigerator Mode | uint4 | - | 0000b – off 0001b – on 0010b – night 0011b – eco 0100b – freezer |