## 6.54 Generic Temperature Control Fan

    - Introduction

The DGN describes a Temperature Control Fan. Temperature Control Fans are installed in power compartments or power distribution centers to improve heat dissipation for one or more power components, or provide warming airflow to components sensitive to cold. A temperature control fan can operate automatically based on temperature thresholds, or turned on/off by remote command. There may be more than one temperature control fan associated with a single power compartment. The following formats apply (see Table 6.54.1).

#### Table 6.54.1 — Temperature Control Fan definition

| Device attribute | Value |
| --- | --- |
| Category | Power components |
| Default Source Address | 151 |
| Dynamic Address Range | 128 to 143 |
| Instance | Multiple |

    - Temperature Control Fan Status

The fan status DGN describes the general operating status of the Temperature Control Fan. Table 6.54.2a defines the DG attributes and Table 6.54.2b defines the signal and parameter attributes.

#### Table 6.54.2a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | TEMPERATURE_CONTROL_FAN_STATUS |
| DGN | 1FD9Bh |
| Default priority | 6 |
| Maximum broadcast rate | 5000 ms |
| Normal broadcast rate | 5000 ms or on change |
| Minimum broadcast rate | 500 ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.54.2b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | see Table 5.3 |
| 1 | 0 to 1 | Operating Mode | uint2 | - | 00b — Automatic 01b — Manual (Temperature Control Fan shall ignore temperature set points) |
| 2 | - | Operating Status (Fan level) | uint8 | % | see Table 5.3 If fan is on/off-only, report 100% or 0%. |
| 3 – 4 | - | Fan Speed | uint16 | - | see Table 5.3 RPMs. 0 = Stopped. 1..65530 = Measured RPMs. |
| 5 – 6 | - | Measured Temperature | uint16 | °C | see Table 5.3 |

    - Temperature Control Fan Command

This command sets the operating mode and starts or stops the Temperature Control Fan. Table 6.xx.10a defines the DG attributes and Table 6.xx.10b defines the signal and parameter attributes.

#### Table 6.54.3a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | TEMPERATURE CONTROL_FAN_COMMAND |
| DGN | 1FD9Ah |
| Default priority | 6 |
| Maximum broadcast rate | N/A |
| Normal broadcast rate | N/A |
| Minimum broadcast rate | N/A |
| Number of frames | 1 |
| ACK requirements | ACK, TEMPERATURE_CONTROL_FAN_STATUS |

Table 6.54.3b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | see Table 5.3 |
| 1 | 0 to 1 | Operating Mode | uint2 | - | 00b — Automatic 01b — Manual (Temperature Control Fan shall ignore temperature set points) |
| 2 | - | Fan Level | uint8 | % | see Table 5.3 If fan is ON/OFF-only, turn ON for any value above 0%. Value is ignored if operating mode is set to automatic. |

    - Temperature Control Fan Configuration Status

This DG provides configuration information for the Temperature Control Fan(s). Table 6.xx.11a defines the DG attributes and Table 6.xx.11b defines the signal and parameter attributes.

#### Table 6.54.4a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | TEMPERATURE_CONTROL_FAN_CONFIGURATION_STATUS |
| DGN | 1FD99h |
| Default priority | 6 |
| Maximum broadcast rate | N/A |
| Normal broadcast rate | on change |
| Minimum broadcast rate | N/A |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.54.4b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value description |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | see Table 5.3 |
| 1 | 0 to 1 | Fan Speed Sensor | uint2 | - | 00b — Fan speed sensor disabled 01b — Fan speed sensor active |
| 2 | 0 to 1 | Temperature Control Mode | uint2 | - | 00b — Cooling 01b — Heating |
| 3 to 4 | - | Set Point Fan Start | uint16 | °C | see Table 5.3 Temperature at which fan starts running. |
| 5 to 6 | - | Set Point Fan Max | uint16 | °C | See Table 5.3 Temperature at which fan is operating at maximum speed. In cooling mode, this set point must be larger or equal to “Set Point Fan Start”. In heating mode, this set point must be less than or equal to “Set Point Fan Start”. |
| 7 | - | Dead band/Hysteresis | uint8 | °C | see Table 5.3 Precision = 0.1 °C Value range = 0.0 to 25.0 °C |

    - Temperature Control Fan Configuration Command

This DGN provides changes in the Temperature Control Fan configuration. Table 6.45.12a defines the DG attributes. The signal and parameter attributes are identical to TEMPERATURE_CONTROL_FAN_CONFIGURATION_STATUS (see Table 6.54.4b). An Instance of Zero indicates that the settings should be applied to all instances. Placing a No Data (255, 65 535) in a field will cause that setting to be ignored. Thus, it is possible to adjust any single setting without changing any others. Unsupported configuration settings for a specific device will be ignored.

#### Table 6.54.5a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | TEMPERATURE_CONTROL_FAN_CONFIGURATION_COMMAND |
| DGN | 1FD98h |
| Default priority | 6 |
| Maximum broadcast rate | N/A |
| Normal broadcast rate | as needed |
| Minimum broadcast rate | N/A |
| Number of frames | 1 |
| ACK requirements | ACK, TEMPERATURE_CONTROL_FAN_CONFIGURATION_STATUS |