# RV-C Specification Submission -

# Water Heater

# Proposed Changes in Red:

6.9 Water Heater

6.9.1 Introduction

The DGs defined in the following clauses support all types of water heaters – typically LP gas, often with electrical elements, and occasionally with diesel burners. Although a single heater is normal, multiple instances are supported. The following formats apply (see Table 6.9.1).

Table 6.9.1 — Water heater definition

| Device attribute | Value |
| --- | --- |
| Category | Appliances |
| Default Source Address | 101, 102 |
| Dynamic Address Range | 208 to 223 |
| Instance | Multi-instance |

6.9.2 Water Heater Status

This DG provides the general water heater status. Table 6.9.2a defines the DG attributes, and Table 6.9.2b defines the signal and parameter attributes.

Table 6.9.2a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | WATERHEATER_STATUS |
| DGN | 1FFF7h |
| Default priority | 6 |
| Maximum broadcast gap | 5000ms |
| Normal broadcast gap | 2000ms or on change |
| Minimum broadcast gap | 500ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.9.2b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 0 – all 1 to 250 - Instance number |
| 1 | - | Operating modes | uint8 | - | 0 – off 1 – combustion 2 – electric 3 - gas/electric (both) 4 - automatic (electric if available, otherwise combustion 5 - test combustion (forced on) 6 - test electric (forced on) |
| 2 to 3 | - | Set point temperature | uint16 | °C | see Table 5.3 The desired water temperature. |
| 4 to 5 | - | Water temperature | uint16 | °C | see Table 5.3 The actual water temperature. |
| 6 | 0 to 1 | Thermostat status | uint2 | - | 00b - set point met 01b - set point not met (heat is being applied) |
|  | 2 to 3 | Burner status | uint2 | - | 00b – off 01b – burner is lit |
|  | 4 to 5 | AC element status | uint2 | - | 00b – AC element is inactive 01b - AC element is active |
|  | 6 to 7 | High temperature limit switch status | uint2 | - | 00b - limit switch not tripped 01b - limit switch tripped |
| 7 | 0 to 1 | Failure to ignite status | uint2 | - | 00b - no failure 01b - device has failed to ignite |
|  | 2 to 3 | AC power failure status | uint2 | - | 00b - AC power present 01b - AC power not present |
|  | 4 to 5 | DC power failure status | uint2 | - | 00b - DC power present 01b - DC power not present |
|  | 6 to 7 | DC power warning status | uint2 | - | 00b - DC power sufficient 01b - DC power warning |

6.9.3 Water Heater Command

This DGN provides external control of the water heater. Table 6.9.3a defines the DG attributes, and Table 6.9.3b defines the signal and parameter attributes.

An instance of zero indicates that the settings should be applied to all water heater instances. Values of 255 (or 65535) indicate that the particular datum should not be changed.

Table 6.9.3a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | WATERHEATER_COMMAND |
| DGN | 1FFF6h |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | NAK, WATERHEATER_STATUS |

Table 6.9.3b — Signal— Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 0 – all 1 to 250 - Instance number |
| 1 | - | Operating modes | uint8 | - | 0 – off 1 – combustion 2 – electric 3 - gas/electric (both) 4 - automatic (electric if available, otherwise combustion 5 - test combustion (forced on) 6 - test electric (forced on) |
| 2 to 3 | - | Set point temperature | uint16 | °C | see Table 5.3 The desired water temperature. |
| 6 | 0 to 3 | Electric Element Level | uint4 | - | Desired Level (1 to 13). Must be less than or equal to the maximum level available. Does not change the operating mode. |

6.9.4 Water Heater Status 2

This DG provides the general water heater status. Table 6.9.4a defines the DG attributes, and Table 6.9.4b defines the signal and parameter attributes.

Table 6.9.4a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | WATERHEATER_STATUS_2 |
| DGN | 1FE99h |
| Default priority | 6 |
| Maximum broadcast gap | 5000 ms |
| Normal broadcast gap | 2000 ms or on change |
| Minimum broadcast gap | 500 ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.9.4b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 0 – all 1 to 250 - Instance number |
| 1 | 0 to 3 | Electric Element Level | uint4 | - | Desired Level (1 to 13) |
|  | 4 to 7 | Max Electric Element Level | uint4 | - | Desired Level (1 to 13). Typically 1 (single element) or 2 (dual low/high element) |
| 2 | 0 to 3 | Engine Preheat | uint4 | - | 0000b - Off 0001b - On 0101b - Test (Forced On) |
|  | 4 to 5 | Coolant Level Warning | uint2 | - | 00b - Coolant level sufficient 01b - Coolant level low. |
|  | 6 to 7 | Hot Water Priority | uint2 | - | 00b - Domestic water priority 01b - Heating priority |
| 3 | 0 to 1 | Output Status:Burner | uint2 | - | 00b - Off 01b - On |
|  | 2 to 3 | Output Status:Burner Indicator | uint2 | - | 00b - Off 01b - On |
|  | 4 to 5 | Output Status:Electric Low | uint2 | - | 00b - Off 01b - On |
|  | 6 to 7 | Output Status:Electric High | uint2 | - | 00b - Off 01b - On |
| 4 | 0 to 1 | Burner Overcurrent Status | uint2 | - | 00b - No overcurrent detected 01b - Overcurrent detected |
|  | 2 to 3 | Burner Undercurrent Status | uint2 | - | 00b - No undercurrent detected 01b - Undercurrent detected |
|  | 4 to 5 | Burner Temperature Status | uint2 | - | 00b - Temperature normal 01b - Temperature warning |
|  | 6 to 7 | Burner Input Status | uint2 | - | 00b - Off (Inactive) 01b - On (Active) |
| 5 | 0 to 1 | Burner Indicator Overcurrent Status | uint2 | - | 00b - No overcurrent detected 01b - Overcurrent detected |
|  | 2 to 3 | Burner Indicator Undercurrent Status | uint2 | - | 00b - No undercurrent detected 01b - Undercurrent detected |
|  | 4 to 5 | Burner Indicator Temperature Status | uint2 | - | 00b - Temperature normal 01b - Temperature warning |
| 6 | 0 to 1 | Electric Low Element Overcurrent Status | uint2 | - | 00b - No overcurrent detected 01b - Overcurrent detected |
|  | 2 to 3 | Electric Low Element Undercurrent Status | uint2 | - | 00b - No undercurrent detected 01b - Undercurrent detected |
|  | 4 to 5 | Electric Low Element Temperature Status | uint2 | - | 00b - Temperature normal 01b - Temperature warning |
|  | 6 to 7 | Electric Low Element Input Status | uint2 | - | 00b - Off (Inactive) 01b - On (Active) |
| 7 | 0 to 1 | Electric High Element Overcurrent Status | uint2 | - | 00b - No overcurrent detected 01b - Overcurrent detected |
|  | 2 to 3 | Electric High Element Undercurrent Status | uint2 | - | 00b - No undercurrent detected 01b - Undercurrent detected |
|  | 4 to 5 | Electric High Element Temperature Status | uint2 | - | 00b - Temperature normal 01b - Temperature warning |
|  | 6 to 7 | Electric High Element Input Status | uint2 | - | 00b - Off (Inactive) 01b - On (Active) |

6.9.5 Water Heater Command 2

This DGN provides external control of the water heater. Table 6.9.5a defines the DG attributes. The signals and attributes are identical to WATERHEATER_STATUS (see Table 6.9.2b).

Table 6.9.5a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | WATERHEATER_COMMAND_2 |
| DGN | 1FE98h |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | NAK, WATERHEATER_STATUS_2 |

Table 6.9.5b — Signal— Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 0 – all 1 to 250 - Instance number |
| 1 | 0 to 3 | Engine Preheat | uint4 | - | 0000b - Off 0001b - On 0101b - Test (Forced On) |
| 2 |  | Command | uint8 | - | 0 = Electric Low - Enable 1 = Electric Low - Disable 2 = Electric Low - Toggle 3 = Electric High - Enable 4 = Electric High - Disable 5 = Electric High - Toggle 6 = Burner - Enable 7 = Burner - Disable 8 = Burner - Toggle 9 = Electric - Cycle (low to high to off) 10 = Electric - Cycle (high to low to off) 11 = Electric Low Test - Toggle 12 = Electric High Test - Toggle 13 = Burner Test - Toggle |
| 3 | 0 to 1 | Hot Water Priority | uint2 | - | For hydronic systems: reduce or disable interior heating to the coach to focus on providing domestic hot water – see “Hot Water Priority” field of WATERHEATER_STATUS_2. Note: some systems may automatically return to normal operation after a set time period. This time period may be hard coded, or may be adjustable with WATERHEATER_CONFIGURATION_COMMAND. 0 – Domestic water priority (disable/reduce interior heating) 1 – Heating priority (Normal system operation) |

6.9.6 Circulation Pump Status

This DGN allows networked devices to display and troubleshoot circulation pumps in a hydronic heating system. The DG format assumes a single hydronic system is installed, and the instance of the pump is independent of the instance of the water heater. Table 6.9.6a defines the DG attributes, and Table 6.9.6b defines the signal and parameter attributes.

Table 6.9.6a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | CIRCULATION_PUMP_STATUS |
| DGN | 1FE97h |
| Default priority | 6 |
| Maximum broadcast gap | 5000 ms |
| Normal broadcast gap | 2000 ms or on change |
| Minimum broadcast gap | 500 ms |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.9.6b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 0 – all 1 to 250 - Instance number |
| 1 | 0 to 3 | Output Status | uint4 | - | 0000b - Off 0001b - On 0101b - Test (Forced On) |
| 2 | 0 to 1 | Pump Overcurrent Status | bit | - | 00b - No overcurrent detected 01b - Overcurrent detected |
|  | 2 to 3 | Pump Undercurrent Status | bit | - | 00b - No undercurrent detected 01b - Undercurrent detected |
|  | 4 to 5 | Pump Temperature Status | bit | - | 00b - Temperature normal 01b - Temperature warning |

6.9.7 Circulation Pump Command

The DG format assumes a single hydronic system is installed, and the instance of the pump is independent of the instance of the water heater. Table 6.9.7a defines the DG attributes, and Table 6.9.7b defines the signal and parameter attributes.

Table 6.9.7a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | CIRCULATION_PUMP_COMMAND |
| DGN | 1FE96h |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | NAK, CIRCULATION_PUMP_STATUS |

Table 6.9.7b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 0 – all 1 to 250 - Instance number |
| 1 | 0 to 3 | Output Mode | uint4 | - | 0000b - Off (End of Test) 0101b - Test (Forced On) |

6.9.10 Circulation Pump Timer Status

This DG allows networked devices to display the time remaining for a given circulation pump’s “test” or override mode in a hydronic heating system, if applicable. Note that this is the test mode initiated using CIRCULATION_PUMP_COMMAND, above. The DG format assumes a single hydronic system is installed, and the instance of the pump is independent of the instance of the water heater. Table 6.9.10a defines the DG attributes, and Table 6.9.10b defines the signal and parameter attributes.

Table 6.9.10a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | CIRCULATION_PUMP_TIMER_STATUS |
| DGN | 1FD9F |
| Default priority | 6 |
| Maximum broadcast gap | None |
| Normal broadcast gap | 1000ms when a timer is active; 5000ms otherwise |
| Minimum broadcast gap | As needed |
| Number of frames | 1 |
| ACK requirements | None |

Table 6.9.10b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 0 – Invalid 1 to 250 - Instance number |
| 1 to 2 | - | Pump override timer | uint16 | s | Time in seconds until pump override/test mode ends (see CIRCULATION_PUMP_COMMAND). 0 – No delay/expired (timer has completed |

6.9.11 Water Heater Timer Status

This DGN allows the water heater to report the time remaining for certain operational modes. Table 6.9.11a defines the DG attributes, and Table 6.9.11b defines the signal and parameter attributes.

Table 6.9.11a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | WATERHEATER_TIMER_STATUS |
| DGN | 1FD9E |
| Default priority | 6 |
| Maximum broadcast gap | None |
| Normal broadcast gap | 1000ms when a timer is active; 5000ms otherwise |
| Minimum broadcast gap | As needed |
| ACK requirements | None |

Table 6.9.11b — Signal— Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 0 – Invalid 1 to 250 - Instance number |
| 1 to 2 | - | System timer | uint16 | s | Time in seconds until heater/element shuts off. 0 – No delay/expired (timer has completed |
| 3 to 4 | - | Domestic water timer | uint16 | s | Time in seconds until water priority returns to default (see “Hot Water Priority” field of WATERHEATER_STATUS_2) 0 – No delay/expired (timer has completed) |

6.9.12 Water Heater Configuration Status

This DGN describes configuration information for the water heater. Table 6.9.12a defines the DG attributes, and Table 6.9.12b defines the signal and parameter attributes.

Table 6.9.12a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | WATERHEATER_CONFIGURATION_STATUS |
| DGN | 1FD9D |
| Default priority | 6 |
| Maximum broadcast gap | None |
| Normal broadcast gap | On change or request. |
| Minimum broadcast gap | As needed |
| ACK requirements | None |

Table 6.9.12b — Signal— Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 0 – Invalid 1 to 250 - Instance number |
| 1 to 2 | - | System timer – starting value | uint16 | s | Length (starting value/time) of system timer (bytes 1-2 of WATERHEATER_TIMER_STATUS) in seconds. 0 – System timer is supported by water heater, but currently disabled (heater/element will run indefinitely). 1 - 65533 – Time in seconds for timer to start at. |
| 3 to 4 | - | Domestic water timer – starting value | uint16 | s | Length (starting value/time) of domestic water timer (bytes 3-4 of WATERHEATER_TIMER_STATUS) in minutes. 0 – Domestic water timer is supported by water heater, but currently disabled (domestic water priority will stay enabled indefinitely). 1 - 65533 – Time in seconds for timer to start at. |

6.9.13 Water Heater Configuration Command

This DGN allows changes in water heater configuration. Table 6.9.X3a defines the DG attributes. The signal and parameter attributes have the same format as WATERHEATER_CONFIGURATION_STATUS (see Table 6.9.X2b).

Table 6.9.13a — DG definition

| DG attribute | Value |
| --- | --- |
| Name | WATERHEATER_CONFIGURATION_COMMAND |
| DGN | 1FD9C |
| Default priority | 6 |
| Maximum broadcast gap | N/A |
| Normal broadcast gap | As needed |
| Minimum broadcast gap | As needed |
| ACK requirements | None |

Table 6.9.13b — Signal— Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 0 – Invalid 1 to 250 - Instance number |
| 1 to 2 | - | System timer – set starting value | uint16 | s | Length (starting value/time) of system timer (bytes 1-2 of WATERHEATER_TIMER_STATUS) in seconds. 0 – Disable system timer (heater/element will run indefinitely). 1 - 65533 – Time in seconds for timer to start at. |
| 3 to 4 | - | Domestic water timer – set starting value | uint16 | s | Length (starting value/time) of domestic water timer (bytes 3-4 of WATERHEATER_TIMER_STATUS) in minutes. 0 – Disable domestic water timer (domestic water priority will stay enabled indefinitely). 1 - 65533 – Time in seconds for timer to start at. |