Justification:

Clarify that NAK 7, “Command requires more time to execute”, does not need to be sent on a regular schedule.

Original Section:

7.5 Standard acknowledgments

Table 7.5 — Standard acknowledgments

| Value | Description |
| --- | --- |
| 0 | ACK. Command will be executed. |
| 1 | NAK. Command will not be executed. |
| 2 | Command is not acceptable from the source. |
| 3 | Conditions do not allow command to be executed. |
| 4 | Command is not formatted properly. |
| 5 | Command parameters are out of range. |
| 6 | Command requires a security password. |
| 7 | Command requires more time to execute. This should be followed by an ACK when complete. |
| 8 | Command overridden by user. |
| 9 to 127 | Reserved |
| 128 to 254 | Command-specific responses. |

These are the valid values for the Acknowledgment field in the Acknowledgment DGN (59392).

NAK 7 is used for operations in which an immediate positive or negative response is not possible. For example, if the node must check the status of other devices before it knows whether the command is possible, and those devices do not respond within the 1250 ms time window allowed for commands, it should send an immediate NAK 7 then send a second message when the outcome is fully known.

Proposed Changes in RED:

7.5 Standard acknowledgments

Table 7.5 — Standard acknowledgments

| Value | Description |
| --- | --- |
| 0 | ACK. Command will be executed. |
| 1 | NAK. Command will not be executed. |
| 2 | Command is not acceptable from the source. |
| 3 | Conditions do not allow command to be executed. |
| 4 | Command is not formatted properly. |
| 5 | Command parameters are out of range. |
| 6 | Command requires a security password. |
| 7 | Command requires more time to execute. This should be followed by an ACK when complete. |
| 8 | Command overridden by user. |
| 9 to 127 | Reserved |
| 128 to 254 | Command-specific responses. |

These are the valid values for the Acknowledgment field in the Acknowledgment DGN (59392).

NAK 7 is used for operations in which an immediate positive or negative response is not possible. For example, if the node must check the status of other devices before it knows whether the command is possible, but those devices do not respond within the 1250 ms time window allowed for commands, it should send an immediate NAK 7 and then send a second message when the outcome is fully known. No periodic broadcast of NAK 7 is required to be sent to indicate a pending response.

It is recommended to always send a NAK 7 if a command action is delayed for any reason other than processing time, even if less than 1250ms.