JUSTIFICATION:

Typos in the Signal and Parameter Definition Table for 7.17.7 & 8, and removal of redundant information since “out-of-range” and “No-data-available” values are default.

Changes Marked in Red and Strike-Through:

Table 6.17.7b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | See Table 5.3 The air conditioner (main, second, third, etc.) containing the component being reported on. Corresponds to “Zones” in user terminology. |
| 1 | - | Secondary Instance | uint8 | - | See Table 6.17.7c The component inside the air conditioner that is being reported on. |
| 2 | 0-3 | Operating direction/mode | uint4 | - | Note: if byte 3 (Output level) is 0%, operating direction/mode is OFF (for compressors, compressor is OFF and reversing valve is in any direction) and the contents of this field can be ignored. 0 – Forward For compressors: compressor is ON and reversing valve is in “COOL” direction For single-direction devices: ON (e.g. a heat strip or non-reversible fan) 1 – Reverse For compressors: compressor is ON and reversing valve is in “HEAT” direction. Does not apply to single-direction devices 2 – 5 13 – Open for future development 6 – Out of Range 7 – No data Available |
|  | 4-5 | De-icing status | uint2 | - | 0 – Not de-icing 1 – De-icing Applies to compressors only. |
| 3 | - | Output Level | uint8 | % | See Table 5.3 Actual output level (not desired or goal output level) |
| 4 | 0-3 | Output Resolution | uint8 | % | 0 – Full resolution (steps of 0.5%) 1 – Single speed (0% or 100%) 2 – Two speed (0%, 50%, or 100%) 3 – Three speed (0%, 33%, 66%, or 100%) … 10 – Ten speed 11 – 13 – Open for future development 14 – Out of range 15 – Data not available |
|  | 4-7 | A/C Component State |  |  | 0 – Off 1 – Delay on starting 2 – Start-up sequence (ramp up) 3 – Running 4 – Delay to stop 5 – Shut-down sequence (ramp down) 6 – Halted – Check status DGNs (e.g. coil freeze) 7 – Disabled due to fault – Check DM_RV 8 – 13 – Open for future development 14 – Error 15 – Data not available |
| 5 | - | Delay Timer | uint8 | % | Delay timer count (e.g. time left until the unit attempts to turn on a fan). 0 – No delay/expired (timer has been completed) 1-240 – Delay timer in seconds (up to 240 seconds) 241 – 5 minutes remaining 242 – 6 minutes remaining … 250 – 14 minutes remaining 254 – Error 255 – No timer/no data. When counting down a timer longer than 240 seconds, the A/C unit/component should count down in 1 minute increments until the timer reaches 4 minutes (240 seconds). The component should then count down in 1 second increments. |

Table 6.17.8b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | See Table 5.3 The air conditioner (main, second, third, etc.) containing the component being commanded. Corresponds to “Zones” in user terminology. |
| 1 | - | Secondary Instance | uint8 | - | See Table 6.17.7c The component inside the air conditioner that is being commanded. Note: a command to one component may cause changes in another component. For example, when the compressor is turned on an A/C may automatically turn on a fan as well as a self-protection feature. |
| 2 | 0-3 | Operating direction/mode | uint4 | - | Note: if byte 3 (Output level) is 0%, desired operating direction/mode is OFF (for compressors, compressor is OFF and reversing valve is in any direction) and the contents of this field can be ignored. 0 – Forward For compressors: turn compressor ON with reversing valve in “COOL” direction For single-direction devices: ON (e.g. a heat strip or non-reversible fan) 1 – Reverse For compressors: turn compressor ON with reversing valve in “HEAT” direction Does not apply to single-direction devices 2 - 5 – Open for future development 6 – Out of Range 7 – No data Available |
|  | 4-5 | Reserved |  |  |  |
| 3 | - | Output Level | uint8 | % | See Table 5.3 |
| 4 | - | Reserved |  |  | Desired component output level. |
| 5 | - | Reserved |  |  |  |