Submission:  DGN shall be required to be used in a manner consistent with their security categories.

Justification:
The WRV-C security model requires the user/installer grant permission to wireless devices to access specific function categories.  For example, a keyfob to start the generator must be approved by the user/installer to access the Generator function category.

If any device - wired or wireless - has a practical effect different than its nominal effect, it creates a backdoor for malicious exploitation.  For example, suppose a vendor chooses to implement a door lock mechanism by hooking up a solenoid to a pair of DC loads, which are controlled via the DC_LOAD_COMMAND DGN.  This would allow a device that is approved only in the Electrical category to take control of a Personal Security function.

Amendment: New Section in 4.4.1

 4.4.1.8 
Practical effect
The practical effect of all DGNs as implemented in the RV shall be consistent with the permission categories of that DGN, as listed in Table 7.2 and 7.7.
For example, the DC_LOAD_COMMAND is categorized as "Electrical".  This command shall not be used to control a moving device, as that function would normally require a DGN in the "Moving Devices" category.   In such a circumstance, the device should use the appropriate DGNs for the actual function, and if no appropriate DGNs exist then new DGNs should be submitted.  

Note:  GENERIC_INDICATOR_COMMAND, GENERIC_INDICATOR_STATUS,  DIGITAL_INPUT_STATUS are given the "RV Model-Specific Control" category.  These DGNs may be applied to all categories except 
"Sensitive Data", "Full Network Access", "Network Security", and "Personal Security".
Additional Columns to Section 7, Tables 7.2 and 7.7
Table 7.2 Default Source Addresses
	Product
	DSA
	Preferred Dynamic Address Range
	Notes
Dynamic Address Group
	Section
	Transmit
	Receive

	Reserved
	0-63
	
	Reserved for reporting generic alarms. 
	6.47.1
	
	

	Multifunction
	See Above
	112-127
	Multifunction
	
	
	

	Generator
	64
	128-143
	Power Components
	6.18
	Generator
	Incidental

	Genstart Controller
	65
	128-143
	Power Components
	6.35
	Generator
	Incidental

	Inverter #1-2
	66, 67
	128-143
	Power Components
	6.19
	Electrical
	Incidental

	Control Panel*
	68
	144-159
	Controls and Displays
	
	Incidental
	Incidental

	Battery State of Charge Monitor
	69
	160-175
	Sensors
	6.49
	Electrical
	Incidental

	Battery
	70
	128-143
	Power Components
	6.49
	Electrical
	Incidental

	Chassis Battery S.O.C.
	71
	160-175
	Sensors
	6.5
	Chassis
	Incidental

	Water/Waste Tank System
	72
	160-175
	Sensors
	6.28
	Incidental
	Incidental

	LPG Tank System
	73
	160-175
	Sensors
	6.28
	Incidental
	Incidental

	Converter #1
	74
	128-143
	Power Components
	6.20
	Electrical
	Incidental

	Converter #2
	75
	128-143
	Power Components
	6.20
	Electrical
	Incidental

	Charge Controller
	76
	128-143
	Power Components
	6.20
	Electrical
	Incidental

	AC Load Monitor / Controller
	77
	128-143
	Power Components
	6.22
	Electrical
	Incidental

	AC Fault Protection System
	78
	128-143
	Power Components
	6.32
	Electrical
	Incidental

	Transfer Switch
	79
	128-143
	Power Components
	6.32
	Electrical
	Incidental

	Weather Station
	80
	160-175
	Sensors
	6.33
	Incidental
	Incidental

	Hydraulic/Electric Leveling System Controller
	81
	176-191
	Mechanical Components
	6.13
	Moving Devices
	Incidental

	Hydraulic/Electric Leveling System
	82
	176-191
	Mechanical Components
	6.13
	Moving Devices
	Incidental

	Air Leveling System
	83
	176-191
	Mechanical Components
	6.13
	Moving Devices
	Incidental

	Slide Room #1-4
	84-87
	176-191
	Mechanical Components
	6.14
	Moving Devices
	Incidental

	Main Thermostat
	88
	192-207
	Comfort Systems
	6.16
	Electrical
	Incidental

	Bedroom Thermostat
	89
	192-207
	Comfort Systems
	6.16
	Electrical
	Incidental

	Thermostat #3-6
	90-93
	192-207
	Comfort Systems
	6.16
	Electrical
	Incidental

	Main Furnace (Conventional)
	94
	192-207
	Comfort Systems
	6.15
	Electrical
	Incidental

	Conventional Furnace #2-3
	95, 96
	192-207
	Comfort Systems
	6.15
	Electrical
	Incidental

	Aux. Heat (e.g Tile Heat), #1-3
	97-99
	192-207
	Comfort Systems
	6.36
	Electrical
	Incidental

	Furnace (Hydronic)
	100
	192-207
	Comfort Systems
	6.15
	Electrical
	Incidental

	Water Heater #1-2
	101, 102
	208-223
	Appliances and Bridges
	6.9
	Plumbing
	Incidental

	Air Conditioners #1-4
	103-106
	192-207
	Comfort Systems
	6.17
	Electrical
	Incidental

	Refrigerator
	107
	208-223
	Appliances and Bridges
	
	Appliances
	Incidental

	Aux. Refrigerator(Wine Cooler)
	108
	208-223
	Appliances and Bridges
	
	Incidental
	Incidental

	Aux. Freezer
	109
	208-223
	Appliances and Bridges
	
	Incidental
	Incidental

	Ice Maker
	110
	208-223
	Appliances and Bridges
	
	Incidental
	Incidental

	Stove
	111
	208-223
	Appliances and Bridges
	
	Incidental
	Incidental

	Audio Entertainment #1-3
	112-114
	224-239
	Entertainment Systems
	
	Incidental
	Incidental

	Video Entertainment #1-3
	115-117
	224-239
	Entertainment Systems
	
	Incidental
	Incidental

	TV Lift (Living, Bedroom)
	118, 119
	224-239
	Entertainment Systems
	
	Moving Devices
	Incidental

	Gas Detectors
	120-125
	208-223
	Appliances and Bridges
	6.10
	Personal Security
	Incidental

	Active Air Suspension
	126
	144-159
	Chassis
	6.12
	Moving Devices
	Incidental

	Water Pump
	127
	208-223
	Appliances and Bridges
	6.29
	Incidental
	Incidental

	Tank Autofill
	128
	208-223
	Appliances and Bridges
	6.30
	Incidental
	Incidental

	Waste Dump
	129
	208-223
	Appliances and Bridges
	6.31
	Incidental
	Incidental

	Awning
	130
	176-191
	Mechanical Components
	6.38
	Moving Devices
	Incidental

	DC Dimmer
	131
	128-143
	Power Components
	6.23, 6.24
	Lighting and Ambience
	Incidental

	DC Input, Keypad
	132
	144-159
	Controls and Displays
	6.25
	Incidental
	Incidental

	Tire Monitor
	133
	160-175
	Sensors
	6.37
	Chassis
	Incidental

	Window Shade Control
	134
	128-143
	Power Components
	6.39
	Personal Security
	Incidental

	Door Control 
	135
	176-191
	Mechanical Components
	6.40
	Personal Security
	Personal Security

	GPS
	136
	160-175
	Sensors
	6.34
	Incidental
	Incidental

	AC Load
	137
	128-143
	Power Components
	6.22
	Electrical
	Incidental

	DC Motor Controller
	138
	128-143
	Power Components
	6.27
	Moving Devices
	Incidental

	DC Disconnect
	139
	128-143
	Power Components
	6.42
	Electrical
	Incidental

	Generic AC Source
	140
	128-143
	Power Components
	6.21
	Electrical
	Incidental

	Solar Charge Controller 
	141
	128-143
	Power Components
	6.45
	Electrical
	Incidental

	Roof Fan
	142
	192-207
	Comfort Systems
	6.46
	Incidental
	Incidental

	External Interface
	143
	208-223
	Appliances and Bridges
	6.44
	Network Security
	Network Security

	Generic Alarm 
	144
	160-175
	Sensors
	6.47
	Incidental
	Incidental

	Window Control 
	145
	176-191
	Mechanical Components
	6.40
	Personal Security
	Incidental

	DC Load
	146
	128-143
	Power Components
	6.23
	Electrical
	Incidental

	Step Control
	147
	176-191
	Mechanical Components
	6.48
	Moving Devices
	Incidental

	Generic Plumbing Valve
	148
	176-191
	Mechanical Components
	
	Plumbing
	Incidental

	Breaker Panel
	149
	128-143
	Power Components
	6.50
	Electrical
	Incidental

	Vehicle Seat
	150
	176-191
	Mechanical Components
	6.41
	Incidental
	Incidental

	Service Tool
	249
	-
	
	
	Network Security
	Incidental

	System Clock
	250
	-
	
	6.4
	Incidental
	Incidental

	Data Logger
	251
	-
	
	
	Incidental
	Incidental

	Chassis Bridge
	252
	144-159
	Chassis
	6.11
	Chassis
	Incidental

	Network Bridge
	253
	144-159
	Appliances and Bridges
	6.43
	Network Security
	Network Security


Table 7.7 – DGN designations
	DGN
	Hex
	Decimal
	Section
	Transmit
	Receive

	DATE_TIME_STATUS
	1FFFF
	131071
	6.4.2
	Clock
	Incidental

	SET_DATE_TIME_COMMAND
	1FFFE
	131070
	6.4.3
	Clock
	Incidental

	DC_SOURCE_STATUS_1
	1FFFD
	131069
	6.5.2
	Electrical
	Incidental

	DC_SOURCE_STATUS_2
	1FFFC
	131068
	6.5.3
	Electrical
	Incidental

	DC_SOURCE_STATUS_3
	1FFFB
	131067
	6.5.4
	Electrical
	Incidental

	COMMUNICATION_STATUS_1
	1FFFA
	131066
	6.6.2
	Incidental
	Incidental

	COMMUNICATION_STATUS_2
	1FFF9
	131065
	6.6.3
	Incidental
	Incidental

	COMMUNICATION_STATUS_3
	1FFF8
	131064
	6.6.4
	Incidental
	Incidental

	WATERHEATER_STATUS
	1FFF7
	131063
	6.9.2
	Plumbing
	Incidental

	WATERHEATER_COMMAND
	1FFF6
	131062
	6.9.3
	Plumbing
	Incidental

	GAS_SENSOR_STATUS
	1FFF5
	131061
	6.10.2
	Personal Security
	Incidental

	CHASSIS_MOBILITY_STATUS
	1FFF4
	131060
	6.11.2
	Chassis, Operating Safety
	Incidental

	CHASSIS_MOBILITY_COMMAND
	1FFF3
	131059
	6.11.4
	Chassis, Operating Safety
	Incidental

	AAS_STATUS
	1FFF0
	131056
	6.12.3
	Chassis, Moving Devices
	Incidental

	AAS_COMMAND
	1FFF1
	131057
	6.12.2
	Chassis, Moving Devices
	Incidental

	reserved
	1FFF0
	131056
	
	
	

	AAS_SENSOR_STATUS
	1FFEF
	131055
	6.12.4
	Chassis, Moving Devices
	Incidental

	LEVELING_CONTROL_COMMAND
	1FFEE
	131054
	6.13.3
	Moving Devices
	Incidental

	LEVELING_CONTROL_STATUS
	1FFED
	131053
	6.13.2
	Moving Devices
	Incidental

	LEVELING_JACK_STATUS
	1FFEC
	131052
	6.13.4
	Moving Devices
	Incidental

	LEVELING_SENSOR_STATUS
	1FFEB
	131051
	6.13.5
	Moving Devices
	Incidental

	HYDRAULIC_PUMP_STATUS
	1FFEA
	131050
	6.13.6
	Moving Devices
	Incidental

	LEVELING_AIR_STATUS
	1FFE9
	131049
	6.13.11
	Moving Devices
	Incidental

	SLIDE_STATUS
	1FFE8
	131048
	6.14.1
	Moving Devices
	Incidental

	SLIDE_COMMAND
	1FFE7
	131047
	6.14.2
	Moving Devices
	Incidental

	SLIDE_SENSOR_STATUS
	1FFE6
	131046
	6.14.3
	Moving Devices
	Incidental

	SLIDE_MOTOR_STATUS
	1FFE5
	131045
	6.14.4
	Moving Devices
	Incidental

	FURNACE_STATUS
	1FFE4
	131044
	6.15.2
	Climate
	Incidental

	FURNACE_COMMAND
	1FFE3
	131043
	6.15.3
	Climate
	Incidental

	THERMOSTAT_STATUS_1
	1FFE2
	131042
	6.16.2
	Climate
	Incidental

	AIR_CONDITIONER_STATUS
	1FFE1
	131041
	6.17.5
	Climate
	Incidental

	AIR_CONDITIONER_COMMAND
	1FFE0
	131040
	6.17.3
	Climate
	Incidental

	GENERATOR_AC_STATUS_1
	1FFDF
	131039
	6.18.3
	Generator
	Incidental

	GENERATOR_AC_STATUS_2
	1FFDE
	131038
	6.18.4
	Generator
	Incidental

	GENERATOR_AC_STATUS_3
	1FFDD
	131037
	6.18.5
	Generator
	Incidental

	GENERATOR_STATUS_1
	1FFDC
	131036
	6.18.25
	Generator
	Incidental

	GENERATOR_STATUS_2
	1FFDB
	131035
	6.18.26
	Generator
	Incidental

	GENERATOR_COMMAND
	1FFDA
	131034
	6.18.27
	Generator
	Incidental

	GENERATOR_START_CONFIG_STATUS
	1 FFD9
	131033
	6.18.28
	Generator
	Incidental

	GENERATOR_START_CONFIG_COMMAND
	1 FFD8
	131032
	6.18.29
	Generator
	Incidental

	INVERTER_AC_STATUS_1
	1FFD7
	131031
	6.19.3
	Electrical
	Incidental

	INVERTER_AC_STATUS_2
	1FFD6
	131030
	6.19.4
	Electrical
	Incidental

	INVERTER_AC_STATUS_3
	1FFD5
	131029
	6.19.5
	Electrical
	Incidental

	INVERTER_STATUS
	1FFD4
	131028
	6.19.6
	Electrical
	Incidental

	INVERTER_COMMAND
	1FFD3
	131027
	6.19.9
	Electrical
	Incidental

	INVERTER_CONFIGURATION_STATUS_1
	1FFD2
	131026
	6.19.10
	Electrical
	Incidental

	INVERTER_CONFIGURATION_STATUS_2
	1FFD1
	131025
	6.19.11
	Electrical
	Incidental

	INVERTER_CONFIGURATION_COMMAND_1
	1FFD0
	131024
	6.19.14
	Electrical
	Incidental

	INVERTER_CONFIGURATION_COMMAND_2
	1FFCF
	131023
	6.19.15
	Electrical
	Incidental

	INVERTER_STATISTICS_STATUS
	1FFCE
	131022
	6.19.18
	Electrical
	Incidental

	INVERTER_APS_STATUS
	1FFCD
	131021
	6.19.19
	Electrical
	Incidental

	INVERTER_DCBUS_STATUS
	1FFCC
	131020
	6.19.20
	Electrical
	Incidental

	INVERTER_OPS_STATUS
	1FFCB
	131019
	6.19.21
	Electrical
	Incidental

	CHARGER_AC_STATUS_1
	1FFCA
	131018
	6.20.3
	Electrical
	Incidental

	CHARGER_AC_STATUS_2
	1FFC9
	131017
	6.20.4
	Electrical
	Incidental

	CHARGER_AC_STATUS_3
	1FFC8
	131016
	6.20.5
	Electrical
	Incidental

	CHARGER_STATUS
	1FFC7
	131015
	6.20.6
	Electrical
	Incidental

	CHARGER_CONFIGURATION_STATUS
	1FFC6
	131014
	6.20.10
	Electrical
	Incidental

	CHARGER_COMMAND
	1FFC5
	131013
	6.20.11
	Electrical
	Incidental

	CHARGER_CONFIGURATION_COMMAND
	1FFC4
	131012
	6.20.12
	Electrical
	Incidental

	reserved
	1FFC3
	131011
	
	
	

	CHARGER_APS_STATUS
	1FFC2
	131010
	6.20.23
	Electrical
	Incidental

	CHARGER_DCBUS_STATUS
	1FFC1
	131009
	6.20.24
	Electrical
	Incidental

	CHARGER_OPS_STATUS
	1FFC0
	131008
	6.20.25
	Electrical
	Incidental

	AC_LOAD_STATUS
	1FFBF
	131007
	6.22.2
	Electrical
	Incidental

	AC_LOAD_COMMAND
	1FFBE
	131006
	6.22.4
	Electrical
	Incidental

	DC_LOAD_STATUS
	1FFBD
	131005
	6.23.2
	Electrical
	Incidental

	DC_LOAD_COMMAND
	1FFBC
	131004
	6.23.4
	Electrical
	Incidental

	DC_DIMMER_STATUS_1
	1FFBB
	131003
	6.24.2
	Incidental
	Incidental

	DC_DIMMER_STATUS_2
	1FFBA
	131002
	6.24.3
	Incidental
	Incidental

	DC_DIMMER_COMMAND
	1FFB9
	131001
	6.24.5
	Lighting 
	Incidental

	DIGITAL_INPUT_STATUS
	1FFB8
	131000
	6.25.2
	RV Model-Specific Control
	Incidental

	TANK_STATUS
	1FFB7
	130999
	6.28.2
	Incidental
	Incidental

	TANK_CALIBRATION_COMMAND
	1FFB6
	130998
	6.28.3
	Plumbing
	Incidental

	TANK_GEOMETRY_STATUS
	1FFB5
	130997
	6.28.4
	Plumbing
	Incidental

	TANK_GEOMETRY_COMMAND
	1FFB4
	130996
	6.28.5
	Plumbing
	Incidental

	WATER_PUMP_STATUS
	1FFB3
	130995
	6.29.2
	Incidental
	Incidental

	WATER_PUMP_COMMAND
	1FFB2
	130994
	6.29.3
	Plumbing
	Incidental

	AUTOFILL_STATUS
	1FFB1
	130993
	6.30.2
	Incidental
	Incidental

	AUTOFILL_COMMAND
	1FFB0
	130992
	6.30.3
	Plumbing
	Incidental

	WASTEDUMP_STATUS
	1FFAF
	130991
	6.31.2
	Incidental
	Incidental

	WASTEDUMP_COMMAND
	1FFAE
	130990
	6.31.3
	Plumbing
	Incidental

	ATS_AC_STATUS_1
	1FFAD
	130989
	6.32.2
	Electrical
	Incidental

	ATS_AC_STATUS_2
	1FFAC
	130988
	6.32.2
	Electrical
	Incidental

	ATS_AC_STATUS_3
	1FFAB
	130987
	6.32.2
	Electrical
	Incidental

	ATS_STATUS
	1FFAA
	130986
	6.32.4
	Electrical
	Incidental

	ATS_COMMAND
	1FFA9
	130985
	6.32.5
	Electrical
	Incidental

	reserved
	1FFA8
	130984
	
	
	

	reserved
	1FFA7
	130983
	
	
	

	reserved
	1FFA6
	130982
	
	
	

	WEATHER_STATUS_1
	1FFA5
	130981
	6.33.4
	Incidental
	Incidental

	WEATHER_STATUS_2
	1FFA4
	130980
	6.33.5
	Incidental
	Incidental

	ALTIMETER_STATUS
	1FFA3
	130979
	6.33.6
	Incidental
	Incidental

	ALTIMETER_COMMAND
	1FFA2
	130978
	6.33.7
	Incidental
	Incidental

	WEATHER_CALIBRATE_COMMAND
	1FFA1
	130977
	6.33.8
	Incidental
	Incidental

	COMPASS_BEARING_STATUS
	1FFA0
	130976
	6.34.2
	Incidental
	Incidental

	COMPASS_CALIBRATE_COMMAND
	1FF9F
	130975
	6.34.3
	Incidental
	Incidental

	reserved (formerly BRIDGE_COMMAND)
	1FF9E
	130974
	
	
	

	reserved (formerly BRIDGE_DGN_LIST)
	1FF9D
	130973
	
	
	

	THERMOSTAT_AMBIENT_STATUS
	1FF9C
	130972
	6.16.11
	Climate
	Incidental

	HEAT_PUMP_STATUS
	1FF9B
	130971
	6.17.4
	Climate
	Incidental

	HEAT_PUMP_COMMAND
	1FF9A
	130970
	6.17.5
	Climate
	Incidental

	CHARGER_EQUALIZATION_STATUS
	1FF99
	130969
	6.20.20
	Electrical
	Incidental

	CHARGER_EQUALIZATION_CONFIGURATION_STATUS
	1FF98
	130968
	6.20.21
	Electrical
	Incidental

	CHARGER_EQUALIZATION_CONFIGURATION_COMMAND
	1FF97
	130967
	6.20.22
	Electrical
	Incidental

	CHARGER_CONFIGURATION_STATUS_2
	1FF96
	130966
	6.20.13
	Electrical
	Incidental

	CHARGER_CONFIGURATION_COMMAND_2
	1FF95
	130965
	6.20.14
	Electrical
	Incidental

	GENERATOR_AC_STATUS_4
	1FF94
	130964
	6.18.6
	Generator
	Incidental

	GENERATOR_ACFAULT_CONFIGURATION_STATUS_1
	1FF93
	130963
	6.18.7
	Generator
	Incidental

	GENERATOR_ACFAULT_CONFIGURATION_STATUS_2
	1FF92
	130962
	6.18.7
	Generator
	Incidental

	GENERATOR_ACFAULT_CONFIGURATION_COMMAND_1
	1FF91
	130961
	6.18.7
	Generator
	Incidental

	GENERATOR_ACFAULT_CONFIGURATION_COMMAND_2
	1FF90
	130960
	6.18.7
	Generator
	Incidental

	INVERTER_AC_STATUS_4
	1FF8F
	130959
	6.19.6
	Electrical
	Incidental

	INVERTER_ACFAULT_CONFIGURATION_STATUS_1
	1FF8E
	130958
	6.19.7
	Electrical
	Incidental

	INVERTER_ACFAULT_CONFIGURATION_STATUS_2
	1FF8D
	130957
	6.19.7
	Electrical
	Incidental

	INVERTER_ACFAULT_CONFIGURATION_COMMAND_1
	1FF8C
	130956
	6.19.7
	Electrical
	Incidental

	INVERTER_ACFAULT_CONFIGURATION_COMMAND_2
	1FF8B
	130955
	6.19.7
	Electrical
	Incidental

	CHARGER_AC_STATUS_4
	1FF8A
	130954
	6.20.6
	Electrical
	Incidental

	CHARGER_ACFAULT_CONFIGURATION_STATUS_1
	1FF89
	130953
	6.20.7
	Electrical
	Incidental

	CHARGER_ACFAULT_CONFIGURATION_STATUS_2
	1FF88
	130952
	6.20.7
	Electrical
	Incidental

	CHARGER_ACFAULT_CONFIGURATION_COMMAND_1
	1FF87
	130951
	6.20.7
	Electrical
	Incidental

	CHARGER_ACFAULT_CONFIGURATION_COMMAND_2
	1FF86
	130950
	6.20.7
	Electrical
	Incidental

	ATS_AC_STATUS_4
	1FF85
	130949
	6.32.2
	Electrical
	Incidental

	ATS_ACFAULT_CONFIGURATION_STATUS_1
	1FF84
	130948
	6.32.3
	Electrical
	Incidental

	ATS_ACFAULT_CONFIGURATION_STATUS_2
	1FF83
	130947
	6.32.3
	Electrical
	Incidental

	ATS_ACFAULT_CONFIGURATION_COMMAND_1
	1FF82
	130946
	6.32.3
	Electrical
	Incidental

	ATS_ACFAULT_CONFIGURATION_COMMAND_2
	1FF81
	130945
	6.32.3
	Electrical
	Incidental

	GENERATOR_DEMAND_STATUS
	1FF80
	130944
	6.35.2
	Generator
	Incidental

	GENERATOR_DEMAND_COMMAND
	1FEFF
	130815
	6.35.3
	Generator
	Incidental

	AGS_CRITERION_STATUS
	1FEFE
	130814
	6.35.4
	Generator
	Incidental

	AGS_CRITERION_COMMAND
	1FEFD
	130813
	6.35.6
	Generator
	Incidental

	FLOOR_HEAT_STATUS
	1FEFC
	130812
	6.36.2
	Climate
	Incidental

	FLOOR_HEAT_COMMAND
	1FEFB
	130811
	6.36.3
	Climate
	Incidental

	THERMOSTAT_STATUS_2
	1FEFA
	130810
	6.16.3
	Climate
	Incidental

	THERMOSTAT_COMMAND_1
	1FEF9
	130809
	6.16.4
	Climate
	Incidental

	THERMOSTAT_COMMAND_2
	1FEF8
	130808
	6.16.5
	Climate
	Incidental

	THERMOSTAT_SCHEDULE_STATUS_1
	1FEF7
	130807
	6.16.7
	Climate
	Incidental

	THERMOSTAT_SCHEDULE_STATUS_2
	1FEF6
	130806
	6.16.8
	Climate
	Incidental

	THERMOSTAT_SCHEDULE_COMMAND_1
	1FEF5
	130805
	6.16.9
	Climate
	Incidental

	THERMOSTAT_SCHEDULE_COMMAND_2
	1FEF4
	130804
	6.16.10
	Climate
	Incidental

	INVERTER_DC_STATUS
	1FEE8
	130792
	6.19.22
	Electrical
	Incidental

	AWNING_STATUS
	1FEF3
	130803
	6.38.2
	Moving Devices
	Incidental

	AWNING_COMMAND
	1FEF2
	130802
	6.38.6
	Moving Devices
	Incidental

	TIRE_RAW_STATUS
	1FEF1
	130801
	6.37.2
	Chassis
	Incidental

	TIRE_STATUS
	1FEF0
	130800
	6.37.3
	Chassis
	Incidental

	TIRE_SLOW_LEAK_ALARM
	1FEEF
	130799
	6.37.4
	Chassis
	Incidental

	TIRE_TEMPERATURE_CONFIGURATION_STATUS
	1FEEE
	130798
	6.37.6
	Chassis
	Incidental

	TIRE_PRESSURE_CONFIGURATION_STATUS
	1FEED
	130797
	6.37.7
	Chassis
	Incidental

	TIRE_PRESSURE_CONFIGURATION_COMMAND
	1FEEC
	130796
	6.37.9
	Chassis
	Incidental

	TIRE_TEMPERATURE_CONFIGURATION_COMMAND
	1FEEB
	130795
	6.37.9
	Chassis
	Incidental

	TIRE_ID_STATUS
	1FEEA
	130794
	6.37.11
	Chassis
	Incidental

	TIRE_ID_COMMAND
	1FEE9
	130793
	6.37.12
	Chassis
	Incidental

	INVERTER_DC_STATUS
	1FEE8
	130792
	6.19.22
	Electrical
	Incidental

	GENERATOR_DEMAND_CONFIGURATION_STATUS (deprecated)
	1FEE7
	130791
	6.35.7
	Generator
	Incidental

	GENERATOR_DEMAND_CONFIGURATION_COMMAND (deprecated)
	1FEE6
	130790
	6.35.8
	Generator
	Incidental

	LOCK_STATUS
	1FEE5
	130789
	6.40.2
	Personal Security
	Personal Security

	LOCK_COMMAND
	1FEE4
	130788
	6.40.3
	Personal Security
	Personal Security

	WINDOW_STATUS
	1FEE3
	130787
	6.40.4
	Personal Security
	Personal Security

	WINDOW_COMMAND
	1FEE2
	130786
	6.40.7.4
	Moving Devices
	Personal Security

	DC_MOTOR_CONTROL_COMMAND
	1FEE1
	130785
	6.27.5
	Moving Devices
	Incidental

	DC_MOTOR_CONTROL_STATUS
	1FEE0
	130784
	6.27.2
	Moving Devices
	Incidental

	WINDOW_SHADE_CONTROL_COMMAND
	1FEDF
	130783
	6.39.4
	Moving Devices
	Incidental

	WINDOW_SHADE_CONTROL_STATUS
	1FEDE
	130782
	6.39.2
	Moving Devices
	Incidental

	AC_LOAD_STATUS_2
	1FEDD
	130781
	6.22.3
	Electrical
	Incidental

	DC_LOAD_STATUS_2
	1FEDC
	130780
	6.23.3
	Electrical
	Incidental

	DC_DIMMER_COMMAND_2
	1FEDB
	130779
	6.24.6
	Lighting and Ambience
	Incidental

	DC_DIMMER_STATUS_3
	1FEDA
	130778
	6.24.4
	Lighting and Ambience
	Incidental

	GENERIC_INDICATOR_COMMAND
	1FED9
	130777
	6.26.3
	RV Model-Specific Control
	Incidental

	GENERIC_CONFIGURATION_STATUS
	1FED8
	130776
	6.3.2
	Incidental
	Incidental

	GENERIC_INDICATOR_STATUS
	1FED7
	130775
	6.26.1.1
	RV Model-Specific Control
	Incidental

	MFG_SPECIFIC_CLAIM_REQUEST
	1FED6
	130774
	3.2.4
	Incidental
	Incidental

	AGS_DEMAND_CONFIGURATION_STATUS
	1FED5
	130773
	6.35.7
	Generator
	Incidental

	AGS_DEMAND_CONFIGURATION_COMMAND
	1FED4
	130772
	6.35.8
	Generator
	Incidental

	DEPRECATED (Prior to 10/31/21, was GPS_STATUS)
	1FED3
	130771
	N/A
	
	

	AGS_CRITERION_STATUS_2
	1FED2
	130770
	6.35.5
	Generator
	Incidental

	SUSPENSION_AIR_PRESSURE_STATUS
	1FED1
	130769
	6.12.5
	Chassis
	Incidental

	DC_DISCONNECT_STATUS
	1FED0
	130768
	6.42.2
	Electrical
	Incidental

	DC_DISCONNECT_COMMAND
	1FECF
	130767
	6.42.3
	Electrical
	Incidental

	INVERTER_CONFIGURATION_STATUS_3
	1FECE
	130766
	6.19.12
	Electrical
	Incidental

	INVERTER_CONFIGURATION_COMMAND_3
	1FECD
	130765
	6.19.16
	Electrical
	Incidental

	CHARGER_CONFIGURATION_STATUS_3
	1FECC
	130764
	6.20.15
	Electrical
	Incidental

	CHARGER_CONFIGURATION_COMMAND_3
	1FECB
	130763
	6.20.16
	Electrical
	Incidental

	DM-RV
	1FECA
	130762
	3.2.5
	Refer to DSA Table
	Refer to DSA Table

	DC_SOURCE_STATUS_4
	1FEC9
	130761
	6.5.5
	Electrical
	Incidental

	DC_SOURCE_STATUS_5
	1FEC8
	130760
	6.5.6
	Electrical
	Incidental

	DC_SOURCE_STATUS_6
	1FEC7
	130759
	6.5.7
	Electrical
	Incidental

	GENERATOR_DC_STATUS_1
	1FEC6
	130758
	6.18.9
	Generator
	Incidental

	GENERATOR_DC_CONFIGURATION_STATUS
	1FEC5
	130757
	6.18.11
	Generator
	Incidental

	GENERATOR_DC_COMMAND
	1FEC4
	130756
	6.18.12
	Generator
	Incidental

	GENERATOR_DC_CONFIGURATION_COMMAND
	1FEC3
	130755
	6.18.13
	Generator
	Incidental

	GENERATOR_DC_EQUALIZATION_STATUS
	1FEC2
	130754
	6.18.22
	Generator
	Incidental

	GENERATOR_DC_EQUALIZATION_CONFIGURATION_STATUS
	1FEC1
	130753
	6.18.23
	Generator
	Incidental

	GENERATOR_DC_EQUALIZATION_CONFIGURATION_COMMAND
	1FEC0
	130752
	6.18.24
	Generator
	Incidental

	CHARGER_CONFIGURATION_STATUS_4
	1FEBF
	130751
	6.20.17
	Electrical
	Incidental

	CHARGER_CONFIGURATION_COMMAND_4
	1FEBE
	130750
	6.20.18
	Electrical
	Incidental

	INVERTER_TEMPERATURE_STATUS
	1FEBD
	130749
	6.19.24
	Electrical
	Incidental

	HYDRAULIC_PUMP_COMMAND
	1FEBC
	130748
	6.13.10
	Moving Devices
	Incidental

	GENERIC_AC_STATUS_1
	1FEBB
	130747
	6.21.2
	Electrical
	Incidental

	GENERIC_AC_STATUS_2
	1FEBA
	130746
	6.21.3
	Electrical
	Incidental

	GENERIC_AC_STATUS_3
	1FEB9
	130745
	6.21.4
	Electrical
	Incidental

	GENERIC_AC_STATUS_4
	1FEB8
	130744
	6.21.5
	Electrical
	Incidental

	GENERIC_ACFAULT_CONFIGURATION_STATUS_1
	1FEB7
	130743
	6.21.8.3
	Electrical
	Incidental

	GENERIC_ACFAULT_CONFIGURATION_STATUS_2
	1FEB6
	130742
	6.21.8.3
	Electrical
	Incidental

	GENERIC_ACFAULT_CONFIGURATION_COMMAND_1
	1FEB5
	130741
	6.21.8.3
	Electrical
	Incidental

	GENERIC_ACFAULT_CONFIGURATION_COMMAND_2
	1FEB4
	130740
	6.21.8.3
	Electrical
	Incidental

	SOLAR_CONTROLLER_STATUS_1
	1FEB3
	130739
	6.45.2
	Electrical
	Incidental

	SOLAR_CONTROLLER_CONFIGURATION_STATUS
	1FEB2
	130738
	6.45.10
	Electrical
	Incidental

	SOLAR_CONTROLLER_COMMAND
	1FEB1
	130737
	6.45.11
	Electrical
	Incidental

	SOLAR_CONTROLLER_CONFIGURATION_COMMAND
	1FEB0
	130736
	6.45.12
	Electrical
	Incidental

	SOLAR_EQUALIZATION_STATUS
	1FEAF
	130735
	6.45.21
	Electrical
	Incidental

	SOLAR_EQUALIZATION_CONFIGURATION_STATUS
	1FEAE
	130734
	6.45.22
	Electrical
	Incidental

	SOLAR_EQUALIZATION_CONFIGURATION_COMMAND
	1FEAD
	130733
	6.45.23
	Electrical
	Incidental

	DC_SOURCE_STATUS_7
	1FEAC
	130732
	6.5.8
	Electrical
	Incidental

	DC_SOURCE_STATUS_8
	1FEAB
	130731
	6.5.9
	Electrical
	Incidental

	DC_SOURCE_STATUS_9
	1FEAA
	130730
	6.5.10
	Electrical
	Incidental

	DC_SOURCE_STATUS_10
	1FEA9
	130729
	6.5.11
	Electrical
	Incidental

	CHASSIS_MOBILITY_STATUS_2
	1FEA8
	130728
	6.11.3
	Chassis
	Incidental

	ROOF_FAN_STATUS_1
	1FEA7
	130727
	6.46.2
	Climate
	Incidental

	ROOF_FAN_COMMAND_1
	1FEA6
	130726
	6.46.4
	Climate
	Incidental

	DC_SOURCE_STATUS_11
	1FEA5
	130725
	6.5.12
	Electrical
	Incidental

	DC_SOURCE_COMMAND
	1FEA4
	130724
	6.5.15
	Electrical
	Incidental

	CHARGER_STATUS_2
	1FEA3
	130723
	6.20.9
	Electrical
	Incidental

	CHARGER_CONFIGURATION_COMMAND_5
	1FEA2
	130722
	6.20.20
	Electrical
	Incidental

	CHARGER_CONFIGURATION_STATUS_5
	1FEA1
	130721
	6.20.19
	Electrical
	Incidental

	GPS_DATE_TIME_STATUS
	1FEA0
	130720
	6.4.4
	Clock
	Incidental

	GENERIC_ALARM_STATUS
	1FE9F
	130719
	6.47.1
	Remote Monitoring
	Remote Monitoring

	GENERIC_ALARM_COMMAND
	1FE9E
	130718
	6.47.2
	Remote Monitoring
	Remote Monitoring

	reserved for deprecation
	1FE9D
	130717
	
	
	

	reserved for deprecation
	1FE9C
	130716
	
	
	

	INVERTER_CONFIGURATION_STATUS_4
	1FE9B
	130715
	6.19.13
	Electrical
	Incidental

	INVERTER_CONFIGURATION_COMMAND_4
	1FE9A
	130714
	6.19.17
	Electrical
	Incidental

	WATERHEATER_STATUS_2
	1FE99
	130713
	6.9.4
	Plumbing
	Incidental

	WATERHEATER_COMMAND_2
	1FE98
	130712
	6.9.5
	Plumbing
	Incidental

	CIRCULATION_PUMP_STATUS
	1FE97
	130711
	6.9.6
	Climate
	Incidental

	CIRCULATION_PUMP_COMMAND
	1FE96
	130710
	6.9.7
	Climate
	Incidental

	BATTERY_STATUS_1
	1FE95
	130709
	6.49.1
	Electrical
	Incidental

	BATTERY_STATUS_2
	1FE94
	130708
	6.49.2
	Electrical
	Incidental

	BATTERY_STATUS_3
	1FE93
	130707
	6.49.3
	Electrical
	Incidental

	BATTERY_STATUS_4
	1FE92
	130706
	6.49.4
	Electrical
	Incidental

	BATTERY_STATUS_5
	1FE91
	130705
	6.49.5
	Electrical
	Incidental

	BATTERY_STATUS_6
	1FE90
	130704
	6.49.6
	Electrical
	Incidental

	BATTERY_STATUS_7
	1FE8F
	130703
	6.49.7
	Electrical
	Incidental

	BATTERY_STATUS_8
	1FE8E
	130702
	6.49.8
	Electrical
	Incidental

	BATTERY_STATUS_9
	1FE8D
	130701
	6.49.9
	Electrical
	Incidental

	BATTERY_STATUS_10
	1FE8C
	130700
	6.49.10
	Electrical
	Incidental

	BATTERY_STATUS_11
	1FE8B
	130699
	6.49.11
	Electrical
	Incidental

	BATTERY_COMMAND
	1FE8A
	130698
	6.49.14
	Electrical
	Incidental

	STEP_STATUS
	1FE89
	130697
	6.48.1
	Moving Devices
	Incidental

	STEP_COMMAND
	1FE88
	130696
	6.48.2
	Moving Devices
	Incidental

	VEHICLE_ENVIRONMENT_STATUS
	1FE87
	130695
	6.11.5
	Operating Safety
	Incidental

	VEHICLE_ENVIRONMENT_COMMAND
	1FE86
	130694
	6.11.6
	Operating Safety
	Incidental

	SOLAR_CONTROLLER_STATUS_2
	1FE85
	130693
	6.45.3
	Electrical
	Incidental

	SOLAR_CONTROLLER_STATUS_3
	1FE84
	130692
	6.45.4
	Electrical
	Incidental

	SOLAR_CONTROLLER_STATUS_4
	1FE83
	130691
	6.45.5
	Electrical
	Incidental

	SOLAR_CONTROLLER_STATUS_5
	1FE82
	130690
	6.45.6
	Electrical
	Incidental

	SOLAR_CONTROLLER_STATUS_6
	1FE81
	130689
	6.45.7
	Electrical
	Incidental

	SOLAR_CONTROLLER_BATTERY_STATUS
	1FE80
	130688
	6.45.8
	Electrical
	Incidental

	SOLAR_CONTROLLER_SOLAR_ARRAY_STATUS
	1FDFF
	130559
	6.45.9
	Electrical
	Incidental

	SOLAR_CONTROLLER_CONFIGURATION_STATUS_2
	1FDFE
	130558
	6.45.13
	Electrical
	Incidental

	SOLAR_CONTROLLER_CONFIGURATION_COMMAND_2
	1FDFD
	130557
	6.45.14
	Electrical
	Incidental

	SOLAR_CONTROLLER_CONFIGURATION_STATUS_3
	1FDFC
	130556
	6.45.15
	Electrical
	Incidental

	SOLAR_CONTROLLER_CONFIGURATION_COMMAND_3
	1FDFB
	130555
	6.45.16
	Electrical
	Incidental

	SOLAR_CONTROLLER_CONFIGURATION_STATUS_4
	1FDFA
	130554
	6.45.17
	Electrical
	Incidental

	SOLAR_CONTROLLER_CONFIGURATION_COMMAND_4
	1FDF9
	130553
	6.45.18
	Electrical
	Incidental

	DC_SOURCE_STATUS_12
	1FDF8
	130552
	6.5.13
	Electrical
	Incidental

	DC_SOURCE_CONFIGURATION_STATUS_1
	1FDF7
	130551
	6.5.16
	Electrical
	Incidental

	DC_SOURCE_CONFIGURATION_COMMAND_1
	1FDF6
	130550
	6.5.17
	Electrical
	Incidental

	DC_SOURCE_CONFIGURATION_STATUS_2
	1FDF5
	130549
	6.5.18
	Electrical
	Incidental

	DC_SOURCE_CONFIGURATION_COMMAND_2
	1FDF4
	130548
	6.5.19
	Electrical
	Incidental

	BATTERY_STATUS_12
	1FDF3
	130547
	6.49.12
	Electrical
	Incidental

	BATTERY_STATUS_13
	1FDF2
	130546
	6.49.13
	Electrical
	Incidental

	BATTERY_SUMMARY
	1FDF1
	130545
	6.49.15
	Electrical
	Incidental

	DEPRECATED (Prior to 103121, was BATTERY_CONFIGURATION_COMMAND_1)
	1FDF0
	130544
	
	
	

	DEPRECATED (Prior to 103121, was BATTERY_CONFIGURATION_STATUS_2)
	1FDEF
	130543
	
	
	

	DEPRECATED (Prior to 103121, was BATTERY_CONFIGURATION_COMMAND_2)
	1FDEE
	130542
	
	
	

	TIRE_HIGH_PRESSURE_CONFIGURATION_STATUS
	1FDED
	130541
	6.37.8
	Chassis
	Incidental

	TIRE_HIGH_PRESSURE_CONFIGURATION_COMMAND
	1FDEC
	130540
	6.37.9
	Chassis
	Incidental

	LEVELING_SENSOR_ROLL_CONFIG_STATUS
	1FDEB
	130539
	6.13.7
	Moving Devices
	Incidental

	LEVELING_SENSOR_ROLL_CONFIG_COMMAND
	1FDEA
	130538
	6.13.8
	Moving Devices
	Incidental

	LEVELING_SENSOR_PITCH_CONFIG_STATUS
	1FDE9
	130537
	6.13.6
	Moving Devices
	Incidental

	LEVELING_SENSOR_PITCH_CONFIG_COMMAND
	1FDE8
	130536
	6.13.8
	Moving Devices
	Incidental

	DC_SOURCE_STATUS_13
	1FDE7
	130535
	6.5.14
	Electrical
	Incidental

	CAN_BUS_STATUS
	1FDE6
	130534
	6.43.1
	Incidental
	Incidental

	VALVE_STATUS
	1FDE5
	130533
	6.51.2
	Plumbing
	Incidental

	VALVE_COMMAND
	1FDE4
	130532
	6.51.3
	Plumbing
	Incidental

	ROOF_FAN_STATUS_2
	1FDE3
	130531
	6.46.3
	Incidental
	Incidental

	ROOF_FAN_COMMAND_2
	1FDE2
	130530
	6.46.5
	Climate
	Incidental

	WEATHER_ALARM_STATUS
	1FDE1
	130529
	6.33.3
	Incidental
	Incidental

	WEATHER_ALARM_COMMAND
	1FDE0
	130528
	6.33.2
	Incidental
	Incidental

	GPS_TIME_STATUS
	1FDDF
	130527
	6.34.6
	Clock
	Incidental

	DC_SOURCE_CONFIGURATION_COMMAND_3
	1FDDE
	130526
	6.5.21
	Electrical
	Incidental

	CELL_DETAIL
	1FDDD
	130525
	6.49.16
	Electrical
	Incidental

	GENERATOR_DC_STATUS_2
	1FDDC
	130524
	6.18.10
	Generator
	Incidental

	GENERATOR_DC_CONFIGURATION_STATUS_2
	1FDDB
	130523
	6.18.14
	Generator
	Incidental

	GENERATOR_DC_CONFIGURATION_COMMAND_2
	1FDDA
	130522
	6.18.15
	Generator
	Incidental

	GENERATOR_DC_CONFIGURATION_STATUS_3
	1FDD9
	130521
	6.18.16
	Generator
	Incidental

	GENERATOR_DC_CONFIGURATION_COMMAND_3
	1FDD8
	130520
	6.18.17
	Generator
	Incidental

	GENERATOR_DC_CONFIGURATION_STATUS_4
	1FDD7
	130519
	6.18.18
	Generator
	Incidental

	GENERATOR_DC_CONFIGURATION_COMMAND_4
	1FDD6
	130518
	6.18.19
	Generator
	Incidental

	DEPRECATED (Prior to 103121, was GENERATOR_DC_CONFIGURATION_STATUS_5)
	1FDD5
	130517
	6.18.20
	
	

	DEPRECATED (Prior to 103121, was GENERATOR_DC_CONFIGURATION_COMMAND_5)
	1FDD4
	130516
	6.18.21
	
	

	REFRIGERATOR_STATUS
	1FDD3
	130515
	6.52.2
	Appliances
	Incidental

	REFRIGERATOR_COMMAND
	1FDD2
	130514
	6.52.3
	Appliances
	Incidental

	DEVICE_STATE_SYNCHRONIZATION
	1FDD1
	130513
	6.2.5
	Synchronization
	Synchronization

	DC_SOURCE_CONNECTION_STATUS
	1FDD0
	130512
	6.5.20
	Electrical
	Incidental

	SOLAR_CONTROLLER_CONFIGURATION_STATUS_5
	1FDCF
	130511
	6.45.19
	Electrical
	Incidental

	SOLAR_CONTROLLER_CONFIGURATION_COMMAND_5
	1FDCE
	130510
	6.45.20
	Electrical
	Incidental

	AWNING_STATUS_2
	1FDCD
	130509
	6.38.4
	Moving Devices
	Incidental

	AWNING_COMMAND_2
	1FDCC
	130508
	6.38.5
	Moving Devices
	Incidental

	INVERTER_TEMPERATURE_STATUS_2
	1FDCB
	130507
	6.19.24
	Electrical
	Incidental

	CHARGER_STATUS_3
	1FDCA
	130506
	6.20.10
	Electrical
	Incidental

	AIR_CONDITIONING_STATUS_2
	1FDC9
	130505
	6.17.3
	Climate
	Incidental

	VEHICLE_SEAT_COMMAND
	1FDC8
	130504
	6.41.2
	Moving Devices
	Incidental

	VEHICLE_SEAT_STATUS
	1FDC7
	130503
	6.41.3
	Incidental
	Incidental

	VEHICLE_SEAT_LIGHTING_COMMAND
	1FDC6
	130502
	6.41.4
	Lighting and Ambience
	Incidental

	VEHICLE_SEAT_LIGHTING_STATUS
	1FDC5
	130501
	6.41.5
	Incidental
	Incidental

	TV_LIFT_STATUS
	1FDC4
	130500
	6.53.2
	Moving Devices
	Incidental

	TV_LIFT_COMMAND
	1FDC3
	130499
	6.53.3
	Moving Devices
	Incidental

	DC_LIGHTING_CONTROLLER_STATUS_1
	1FDC2
	130498
	6.24.9
	Incidental
	Incidental

	DC_LIGHTING_CONTROLLER_STATUS_2
	1FDC1
	130497
	6.24.10
	Incidental
	Incidental

	DC_LIGHTING_CONTROLLER_STATUS_3
	1FDC0
	130496
	6.24.11
	Incidental
	Incidental

	DC_LIGHTING_CONTROLLER_STATUS_4
	1FDBF
	130495
	6.24.12
	Incidental
	Incidental

	DC_LIGHTING_CONTROLLER_STATUS_5
	1FDBE
	130494
	6.24.13
	Incidental
	Incidental

	DC_LIGHTING_CONTROLLER_STATUS_6
	1FDBD
	130493
	6.24.14
	Incidental
	Incidental

	DC_LIGHTING_CONTROLLER_COMMAND_1
	1FDBC
	130492
	6.24.15
	Lighting and Ambience
	Incidental

	DC_LIGHTING_CONTROLLER_COMMAND_2
	1FDBB
	130491
	6.24.16
	Lighting and Ambience
	Incidental

	DC_LIGHTING_CONTROLLER_COMMAND_3
	1FDBA
	130490
	6.24.17
	Lighting and Ambience
	Incidental

	DC_LIGHTING_CONTROLLER_COMMAND_4
	1FDB9
	130489
	6.24.18
	Lighting and Ambience
	Incidental

	DC_LIGHTING_CONTROLLER_COMMAND_5
	1FDB8
	130488
	6.24.19
	Lighting and Ambience
	Incidental

	DC_LIGHTING_CONTROLLER_COMMAND_6
	1FDB7
	130487
	6.24.20
	Lighting and Ambience
	Incidental

	BROKER_STATUS
	1FDB6
	130486
	6.44.1
	Network Security
	Network Security

	BROKER_INITIATOR_COMMAND
	1FDB5
	130485
	6.44.2
	Network Security
	Network Security

	BROKER _INITIATOR_STATUS
	1FDB4
	130484
	6.44.3
	Network Security
	Network Security

	BROKER_RESPONDER_COMMAND
	1FDB3
	130483
	6.44.4
	Network Security
	Network Security

	ROKER _RESPONDER_STATUS
	1FDB2
	130482
	6.44.5
	Network Security
	Network Security

	BROKER_CONNECTION_COMMAND
	1FDB1
	130481
	6.44.6
	Network Security
	Network Security

	BROKER_CONNECTION_STATUS
	1FDB0
	130480
	6.44.7
	Network Security
	Network Security

	BROKER_CONNECTION_TEXT_COMMAND
	1FDAF
	130479
	6.44.8
	Network Security
	Network Security

	BROKER_CONNECTION_TEXT_STATUS
	1FDAE
	130478
	6.44.9
	Network Security
	Network Security

	AIR_CONDITIONER_COMPONENT_STATUS
	1FDAD
	130477
	6.17.7
	Climate
	Incidental

	AIR_CONDITIONER_COMPONENT_COMMAND
	1FDAC
	130476
	6.17.8
	Climate
	Incidental

	AIR_CONDITIONER_COMPONENT_STATUS_2
	1FDAB
	130475
	6.17.9
	Climate
	Incidental

	CHARGER_PROPERTIES
	1FDAA
	130474
	6.20.24
	Electrical
	Incidental

	DC_SOURCE_LOAD_CONTROL
	1FDA9
	130473
	6.5.24
	Electrical
	Incidental

	BATTERY_LOAD_CONTROL
	1FDA8
	130472
	6.49.20
	Electrical
	Incidental

	DC_SOURCE_CONFIGURATION_COMMAND_4
	1FDA7
	130471
	6.5.25
	Electrical
	Incidental

	DC_SOURCE_CONFIGURATION_STATUS_4
	1FDA6
	130470
	6.5.26
	Electrical
	Incidental

	DC_SOURCE_STATUS_14
	1FDA5
	130469
	6.5.27
	Electrical
	Incidental

	DC_SOURCE_STATUS_15
	1FDA4
	130468
	6.5.28
	Electrical
	Incidental

	DC_SOURCE_STATUS_16
	1FDA3
	130467
	6.5.29
	Electrical
	Incidental

	DC_SOURCE_STATUS_17
	1FDA2
	130466
	6.5.30
	Electrical
	Incidental

	FURNACE_CONFIGURATION_STATUS
	1FDA1
	130465
	6.15.6
	Climate
	Incidental

	FURNACE_CONFIGURATION_COMMAND
	1FDA0
	130464
	6.15.7
	Climate
	Incidental

	CIRCULATION_PUMP_TIMER_STATUS
	1FD9F
	130463
	6.9.10
	Climate
	Incidental

	WATERHEATER_TIMER_STATUS
	1FD9E
	130462
	6.9.11
	Plumbing
	Incidental

	WATERHEATER_CONFIGURATION_STATUS
	1FD9D
	130461
	6.9.12
	Plumbing
	Incidental

	WATERHEATER_CONFIGURATION_COMMAND
	1FD9C
	130460
	6.9.13
	Plumbing
	Incidental

	TEMPERATURE_CONTROL_FAN_STATUS
	1FD9B
	130459
	6.54.2
	RV Model-Specific Control
	Incidental

	TEMPERATURE_CONTROL_FAN_COMMAND
	1FD9A
	130458
	6.54.3
	RV Model-Specific Control
	Incidental

	TEMPERATURE_CONTROL_FAN_CONFIGURATION_STATUS
	1FD99
	130457
	6.54.4
	RV Model-Specific Control
	Incidental

	TEMPERATURE_CONTROL_FAN_CONFIGURATION_COMMAND
	1FD98
	130456
	6.54.5
	RV Model-Specific Control
	Incidental

	GENERIC_REVISION_STATUS
	1FD97
	130455
	6.2.6
	Remote Monitoring
	Remote Monitoring

	GENERIC_REVISION_REQUEST
	1FD96
	130454
	6.2.7
	Remote Monitoring
	Remote Monitoring

	GENERAL_RESET
	17F##
	98048 + address
	6.2.1
	Full Network Access
	Incidental

	TERMINAL
	17E##
	97792 + address
	6.2.3
	Full Network Access
	Full Network Access

	DOWNLOAD
	17D##
	97536 + address
	6.2.2
	Full Network Access
	Full Network Access

	INSTANCE_ASSIGNMENT
	17C##
	97280 + address
	6.2.4
	Full Network Access
	Full Network Access

	INSTANCE_STATUS
	17B##
	97024 + address
	6.2.4
	Full Network Access
	Full Network Access

	DC_COMPONENT_DRIVER_STATUS_1
	16F##
	93952 + DSA
	6.50.1
	Full Network Access
	Incidental

	DC_COMPONENT_DRIVER_STATUS_2
	16E##
	93696 + DSA
	6.50.2
	Full Network Access
	Incidental

	DC_COMPONENT_DRIVER_STATUS_3
	16D##
	93440 + DSA
	6.50.3
	Full Network Access
	Incidental

	DC_COMPONENT_DRIVER_STATUS_4
	16C##
	93184 + DSA
	6.50.4
	Full Network Access
	Incidental

	DC_COMPONENT_DRIVER_STATUS_5
	16B##
	92928 + DSA
	6.50.5
	Full Network Access
	Incidental

	DC_COMPONENT_DRIVER_SETTINGS_1
	16A##
	92672 + DSA
	6.50.6
	Full Network Access
	Full Network Access

	DC_COMPONENT_DRIVER_SETTINGS_2
	169##
	92416 + DSA
	6.50.7
	Full Network Access
	Full Network Access

	DC_COMPONENT_DRIVER_COMMAND
	160##
	90112 + DSA
	6.50.10
	Full Network Access
	Full Network Access

	DC_COMPONENT_DRIVER_SETTINGS_COMMAND_1
	161##
	90368 + DSA
	6.50.8
	Full Network Access
	Full Network Access

	DC_COMPONENT_DRIVER_SETTINGS_COMMAND_2
	162##
	90624 + DSA
	6.50.9
	Full Network Access
	Full Network Access

	DC_COMPONENT_DRIVER_STATUS_6
	163##
	90880 + DSA
	6.50.11
	Full Network Access
	Incidental

	GPS_POSITION
	FEF3
	65267
	6.34.4
	
	


