Justification:

Clarification of Definition of table 6.47.1b Generic_Alarm_Status – Alarm for User Notification Definition.

Original:

6.47.1b— Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 1 – 253 |
| 1 |  | DSA | uint8 | - | 0-63 – Generic. 64-253 – DSA of the device function. |
| 2 | 0 to 1 | Alarm Triggered | uint2 | - | 00b – Alarm is not Triggered. Either the condition is “Ok” or it is not Ready. 01b – Alarm is Triggered |
|  | 2 to 3 | Alarm Ready | uint2 | - | 00b – Alarm condition is not being monitored. 01b – Alarm condition is being monitored. |
|  | 4 to 5 | Alarm is Acknowledged | uint2 | - | 00b – Alarm has not been acknowledged 01b – Alarm has been triggered and acknowledged. |
|  | 6 to 7 | Automatic Reset Enabled | uint2 | - | 00b – Alarm will not automatically return to the Ready State. 01b – Alarm will automatically return to the Ready State once an “Ok” condition is observed. |
| 3-4 |  | Elapsed Time | uint16 | min | Time elapsed while triggered. This should not be reset to zero until the alarm is triggered again. |
| 5 |  | Device Instance | uint8 |  | Instance of the specific device generating the Alarm, if necessary. If the device type is multi-instances, FFh = All Instances. If the device is single-instanced, this should always be FFh. |
| 6 | 0 to 1 | Full-Time Monitoring Required | uint2 | - | 00b - Full-Time monitoring of alarm is not required. 01b - Full-Time monitoring of alarm is required. Devices monitoring this alarm shall treat the lack its transmission as an alarm. |
|  | 2 to 3 | Alarm for Logging | uint2 | - | 00b - Do not log alarm now. 01b - Alarm should be logged for statistical or forensic purposes. In general, this flag should be set only on the initial triggering of the alarm. In subsequent reports of the same alarm, this should be set to 00b, until such time as the alarm has reset and triggered again. |
|  | 4 to 5 | Alarm for User Notification | uint2 | - | 00b - Do not notify user. 01b - User notification required. In general, at least one of these two flags - Alarm for Logging and/or Alarm for User Notification, should be set. |
|  | 6 to 7 | Reserved | uint2 | - |  |

Proposed Changes in Red:

6.47.1b— Signal and parameter definition

| Byte | Bit | Name | Data Type | Unit | Definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | Instance | uint8 | - | 1 – 253 |
| 1 |  | DSA | uint8 | - | 0-63 – Generic. 64-253 – DSA of the device function. |
| 2 | 0 to 1 | Alarm Triggered | uint2 | - | 00b – Alarm is not Triggered. Either the condition is “Ok” or it is not Ready. 01b – Alarm is Triggered |
|  | 2 to 3 | Alarm Ready | uint2 | - | 00b – Alarm condition is not being monitored. 01b – Alarm condition is being monitored. |
|  | 4 to 5 | Alarm is Acknowledged | uint2 | - | 00b – Alarm has not been acknowledged 01b – Alarm has been triggered and acknowledged. |
|  | 6 to 7 | Automatic Reset Enabled | uint2 | - | 00b – Alarm will not automatically return to the Ready State. 01b – Alarm will automatically return to the Ready State once an “Ok” condition is observed. |
| 3-4 |  | Elapsed Time | uint16 | min | Time elapsed while triggered. This should not be reset to zero until the alarm is triggered again. |
| 5 |  | Device Instance | uint8 |  | Instance of the specific device generating the Alarm, if necessary. If the device type is multi-instances, FFh = All Instances. If the device is single-instanced, this should always be FFh. |
| 6 | 0 to 1 | Full-Time Monitoring Required | uint2 | - | 00b - Full-Time monitoring of alarm is not required. 01b - Full-Time monitoring of alarm is required. Devices monitoring this alarm shall treat the lack its transmission as an alarm. |
|  | 2 to 3 | Alarm for Logging | uint2 | - | 00b - Do not log alarm now. 01b - Alarm should be logged for statistical or forensic purposes. In general, this flag should be set only on the initial triggering of the alarm. In subsequent reports of the same alarm, this should be set to 00b, until such time as the alarm has reset and triggered again. |
|  | 4 to 5 | Alarm for User Notification | uint2 | - | 00b - Do not notify user. 01b - User notification required. In general, when the alarm is initially triggered, at least one of these two flags - Alarm for Logging and/or Alarm for User Notification, should be set. |
|  | 6 to 7 | Reserved | uint2 | - |  |