Table 6.5.6b – Deprecate Byte 6-7: DC Voltage Rate of Change

Justification:

The description contains a self-contradiction, confusing "time" and "temperature".  Under either interpretation, the measurement is of doubtful value in describing a DC Source and has been implemented rarely, if ever.

Table 6.5.6b — Signal and parameter definition

| Byte | Bit | Name | Data type | Unit | Value definition |
| --- | --- | --- | --- | --- | --- |
| 0 | - | DC Instance | uint8 | - | see table 6.5.2b |
| 1 | - | Device priority | uint8 | - | see table 6.5.2b |
| 2 to 5 | - | HP DC voltage | uint32 | Vdc | Precision = 0.001 V |
| 6 to 7 | - | DC Voltage Rate of Change (dV/dT, dt means change in temperature) DEPRECATED | uint16 | mV/s | Precision = 1 mV/s 0 = -32000 mV/s 32000 = 0 mV/s 65530 = 33530 mV/s (Max) DEPRECATED |